Natural Production Monitoring and Management in the Deschutes Basin

Project Narrative

| Project Name | Natural Production Monitoring and Management |
| --- | --- |
| Project Number | 2008-311-00 |
| Proposer | Confederated Tribes of the Warm Springs Reservation of Oregon (CTWSRO) |
| Short Description | Project Purposes: (1) Continue annual monitoring of natural production of spring Chinook salmon and steelhead in streams on the Warm Springs Indian Reservation, Oregon, (2) implement new projects to increase and improve monitoring technology and methods, and (3) provide co-management support of Deschutes River Basin fisheries. |
| Province(s) | Columbia Plateau |
| Subbasin(s) | Deschutes |
| Contact Name | Jens Lovtang, Brad Houslet |
| Contact email | jlovtang@wstribes.org, bhouslet@wstribes.org |

ABSTRACT

The Warm Springs River and Shitike Creek support naturally spawning spring Chinook salmon Oncorhynchus tshawytscha and summer steelhead O. mykiss, which are integral parts of the cultural and spiritual identity of the people of the Confederated Tribes of the Warm Springs Reservation of Oregon (CTWSRO).  Spring Chinook salmon are the primary species of cultural identity.  Steelhead are included in the Deschutes River Westside population, which is listed as threatened under the Endangered Species Act (ESA).  The Recovery Plan for Mid-Columbia River steelhead and the biological opinion for the Federal Columbia River Power System have identified this population as high priority for improving precision and accuracy of abundance, productivity, diversity and spatial structure information.  Presence of natural-origin fish, status under the ESA, and importance to the CTWSRO are compelling reasons to monitor spring Chinook salmon and steelhead in the Warm Springs River and Shitike Creek.  The CTWSRO has monitored aspects of the natural production of spring Chinook salmon and steelhead in these watersheds for over three decades, beginning with smolt outmigration monitoring on the Warm Springs River in 1977.  Numbers of returning wild spring Chinook salmon and steelhead adults have varied, with a decline between 2006 and 2009.  Adult returns in 2010 were the highest since 2005 for spring Chinook salmon and the highest since 2003 for steelhead.

The goals of this project are to 1) continue annual life stage monitoring of wild spring Chinook salmon and steelhead in the Warm Springs River and Shitike Creek, and (2) provide management and co-management direction of fisheries resources in the Deschutes River Subbasin.  We will focus on Goal 1, through monitoring adult escapement, conducting spawning ground surveys (redd counts), collecting information on juvenile rearing, and estimating the number of juvenile outmigrants.  Although some of the information used to manage fish resources in the Deschutes River Subbasin (Goal 2) will be collected through work conducted as part of this proposal, the weighing of that information by co-managers, and the actual management decisions are not part of this proposal.  These management activities are policy in nature rather than technical.  We therefore limit our description to a general overview of management activities.

A. TECHNICAL AND SCIENTIFIC BACKGROUND

Introduction

The Warm Springs River and Shitike Creek support naturally spawning spring Chinook salmon Oncorhynchus tshawytscha and summer steelhead O. mykiss, from which hatchery-origin fish are generally isolated.  Spring Chinook salmon are the primary species of cultural identity for the people of the Warm Springs Tribes.  All natural production of spring Chinook in the Lower Deschutes River occurs in these two tributaries, and the status and trends of this population is of vital importance to Deschutes Basin fisheries managers.  Steelhead are included in the Deschutes River Westside population, which is a component of the Middle Columbia River Steelhead Distinct Population Segment (DPS).  The DPS is listed as threatened under the Endangered Species Act (ESA).  Presence of natural-origin fish, status under the ESA, and importance to the Confederated Tribes of the Warm Springs Reservation of Oregon (CTWSRO) are compelling reasons to monitor spring Chinook salmon and steelhead.

The CTWSRO has monitored aspects of the natural production of spring Chinook salmon and steelhead in these watersheds for over three decades, beginning with smolt outmigration monitoring on the Warm Springs River in 1977 (Table 1).  Redd counts on index reaches began in 1982 for steelhead and 1986 for spring Chinook salmon.

Due to a high rate of personnel turnover, periodic lack of staff, and competing priorities, many of the monitoring efforts on the CTWSRO during the past three decades have been conducted with varying methodologies and levels of effort that complicate comparison of results among years.  The proposed project will address the need for standardized data collection methods that allow for among-year comparisons over the life of the project and beyond.  This will be accomplished by establishing rigorous data collection and analysis protocols for adult escapement, spawning ground (redd) surveys, summertime rearing (snorkeling) surveys, and juvenile outmigration trapping that will guide future data collection efforts.  A passive integrated transponder (PIT) tagging program will be initiated to provide information about migration timing and return rates of wild populations of spring Chinook salmon and steelhead.

Spring Chinook salmon have been the focus of past monitoring efforts.  Although steelhead are an important component of anadromous fish populations in the Deschutes River, little previous efforts have focused only on steelhead production monitoring.  Data have been collected on juvenile steelhead both at outmigrant traps and during summer snorkel surveys, but little effort has been put into the analysis of this data.  To this end, historic estimates of juvenile outmigration and summertime rearing densities for steelhead are lacking.

The proposed project will continue life-cycle monitoring to maintain annual trend status data for spring Chinook salmon and steelhead populations in Reservation streams.  These monitoring efforts will focus on the life history stages present in Reservation streams (e.g. adult escapement, adult spawning ground [redd] surveys, juvenile rearing, and juvenile outmigration).

Table 1.  Spring Chinook and summer steelhead monitoring activities by the CTWSRO 1977 – 2009.

| Activity |  | Season | Years conducted |  |
| --- | --- | --- | --- | --- |
|  |  |  | Warm Springs River | Shitike Creek |
| Adult spawning escapement |  |  |  |  |
|  | Spring Chinook | Spring/Summer | 1981 - Present (at Warm Springs National Fish Hatchery) | 2004 - Present |
|  | Summer steelhead | Late Winter/Spring | Same | 2004 - 2008 |
| Index site redd counts |  |  |  |  |
|  | Spring Chinook | Late Summer | 1986 - Present | 1986 - present |
|  | Summer steelhead | Spring | 1982 - Present | 1982 - present |
| Juvenile rearing |  |  |  |  |
|  | Spring Chinook | Summer | 2002 - 2004, 2006 - Present | 2002 - 2004, 2006 - present |
|  | Summer steelhead | Same | Same | Same |
| Out-migrant trapping |  |  |  |  |
|  | Spring Chinook | Spring/Fall | 1977 - Present | 1977 - Present |
|  | Summer steelhead | Same | Same | Same |

Study Area

The Warm Springs Indian Reservation is located on the east slope of the Cascade Mountains in Oregon, north and east of Mt. Jefferson (Figure 1).  The Reservation is approximately 660,000 acres in size and contains a large portion of the Deschutes River Subbasin downstream of the Pelton - Round Butte hydro-electric project.  Due to their High Cascade origin, many of the Reservation watersheds have excellent water quantity and quality creating a stronghold for Deschutes Subbasin anadromous and resident fish populations.  Reservation streams provide habitat for migration, spawning, and rearing by anadromous salmonids such as spring Chinook salmon, fall Chinook salmon, and ESA- listed summer steelhead, and resident salmonids such as redband trout and ESA-listed bull trout.

All natural production of spring Chinook salmon in the Deschutes River Subbasin occurs in the Warm Springs River and Shitike Creek.  Summer steelhead spawn and rear in much the same physical locations as spring Chinook salmon in the Warm Springs River watershed and Shitike Creek, but also spawn in the mainstem Deschutes River and other areas of the subbasin.

Figure 1.  Streams and fish species distribution on the reservation of the Confederated Tribes of Warm Springs.

The Warm Springs River watershed covers approximately 1,490 square kilometers, with elevations reaching from nearly 1,850 meters (6,000 feet) in the Cascade Mountains to 375 meters (1,230 feet) at its confluence with the Deschutes River at river kilometer (Rkm) 135.  The river flows approximately 85 kilometers and provides an estimated 66 km of anadromous fish habitat.  Two major tributaries to the Warm Springs River, Mill Creek and Beaver Creek, also support anadromous fish.  Summer base flows in the Warm Springs River average from 250-300 cubic feet per second (cfs), with winter/spring freshets increasing flows to over 3,000 cfs for brief periods.

Shitike Creek drains 197 square kilometers, with elevations ranging from a high of 1,610 meters (5,280 feet) down to 450 meters (1,475 feet) at its confluence with the Deschutes River at Rkm 156.  The Creek flows approximately 48 km and provides approximately 42 km of anadromous fish habitat.  Shitike Creek is a flashy stream typified by annual high water events that occur in the spring.  Late winter/early spring flows average about 80 to 120 cfs, but spring freshets can increase flows to over 1,500 cfs for brief periods.

Monitoring Activities

Adult Escapement

Warm Springs River - Adult fish returns to the Warm Springs River are counted at the Warm Springs National Fish Hatchery (WSNFH), located at Rkm 17, which has a barrier weir that allows for enumeration of all upstream migrating fish, including spring Chinook salmon and summer steelhead.  The U.S. Fish and Wildlife Service (USFWS) operates the WSNFH in cooperation with the CTWSRO.  Annually, the WSNFH collects up to 630 adult spring Chinook salmon for broodstock and produces approximately 750,000 smolts annually for release into the Warm Springs River.  The hatchery does not produce summer steelhead.  Hatchery operations are described in the operational and imple-mentation plan that is updated every five years (CTWRSO and USFWS 2007).  The current plan expires in 2011 and the next version (2012 – 2017) is currently in development.

The majority of natural spawning of spring Chinook salmon and steelhead in the Warm Springs River takes place upstream of the WSNFH.  Only wild (unmarked) spring Chinook salmon are passed upstream of the hatchery.  Hatchery-origin fish are either kept for broodstock or sacrificed and donated to the CTWSRO for consumption.  Run size of wild spring Chinook salmon to the Warm Springs River has varied greatly over the past three decades, from a low of 162 adults in 1995 to a high of over 2,600 in 2000.  Returns of hatchery spring Chinook salmon adults to the WSNFH began in 1981 and have varied from a low of 42 in 1994 to a high of nearly 7,000 in 2002 (Table 2).

The number of wild summer steelhead passed upstream of the WSNFH annually has ranged from 79 to 880, with an average of 327 (Table 3).  Counts of hatchery-origin summer steelhead returning have ranged from 16 to 988, with an average of 233.  Hatchery fish are sacrificed and donated to the tribes for consumption.

Table 2.  Wild and hatchery spring Chinook salmon at the WSNFH, 1975 – 2010.

| Return year | Wild adults | Wild jacks | Hatchery adults | Hatchery jacks | Percent wild adults | Percent wild jacks |
| --- | --- | --- | --- | --- | --- | --- |
| 1975 | 2,182 | 0 | 0 | 0 | 100.0% | -- |
| 1976 | 2,878 | 0 | 0 | 0 | 100.0% | -- |
| 1977 | 1,505 | 101 | 0 | 0 | 100.0% | 100.0% |
| 1978 | 2,584 | 76 | 0 | 0 | 100.0% | 100.0% |
| 1979 | 1,322 | 73 | 0 | 0 | 100.0% | 100.0% |
| 1980 | 968 | 34 | 0 | 0 | 100.0% | 100.0% |
| 1981 | 1,525 | 54 | 85 | 0 | 94.7% | 100.0% |
| 1982 | 1,408 | 46 | 895 | 21 | 61.1% | 68.7% |
| 1983 | 1,523 | 34 | 355 | 16 | 81.1% | 68.0% |
| 1984 | 1,192 | 164 | 789 | 203 | 60.2% | 44.7% |
| 1985 | 1,099 | 56 | 1,073 | 36 | 50.6% | 60.9% |
| 1986 | 1,656 | 55 | 152 | 197 | 91.6% | 21.8% |
| 1987 | 1,697 | 86 | 484 | 258 | 77.8% | 25.0% |
| 1988 | 1,578 | 69 | 431 | 393 | 78.5% | 14.9% |
| 1989 | 1,344 | 65 | 2,362 | 176 | 36.3% | 27.0% |
| 1990 | 1,821 | 46 | 1,324 | 66 | 57.9% | 41.1% |
| 1991 | 773 | 40 | 600 | 44 | 56.3% | 47.6% |
| 1992 | 1,040 | 17 | 771 | 20 | 57.4% | 45.9% |
| 1993 | 528 | 6 | 295 | 14 | 64.2% | 30.0% |
| 1994 | 425 | 10 | 42 | 10 | 91.0% | 50.0% |
| 1995 | 162 | 75 | 216 | 75 | 42.9% | 50.0% |
| 1996 | 1,261 | 26 | 712 | 23 | 63.9% | 53.1% |
| 1997 | 861 | 9 | 1,129 | 9 | 43.3% | 50.0% |
| 1998 | 248 | 23 | 647 | 6 | 27.7% | 79.3% |
| 1999 | 367 | 127 | 2,644 | 126 | 12.2% | 50.2% |
| 2000 | 2,622 | 81 | 6,666 | 79 | 28.2% | 50.6% |
| 2001 | 2,155 | 97 | 4,369 | 97 | 33.0% | 50.0% |
| 2002 | 1,388 | 51 | 6,840 | 51 | 16.9% | 50.0% |
| 2003 | 1,280 | 120 | 5,906 | 117 | 17.8% | 50.6% |
| 2004 | 2,350 | 78 | 3,436 | 108 | 40.6% | 41.9% |
| 2005 | 677 | 30 | 962 | 201 | 41.3% | 13.0% |
| 2006 | 1,015 | 68 | 2,636 | 140 | 27.8% | 32.7% |
| 2007 | 367 | 32 | 1,196 | 366 | 23.5% | 8.0% |
| 2008 | 529 | 24 | 1,896 | 245 | 21.8% | 8.9% |
| 2009 | 421 | 191 | 2,133 | 553 | 16.5% | 25.7% |
| 2010 | 1,557 | 35 | 1,173 | 275 | 57.0% | 11.3% |
| Average | 1,286 | 58 | 1,450 | 109 | 59.6% | 49.1% |

Table 3.  Wild and Hatchery Steelhead at the WSNFH, 1977 – 2010.

| Return year | Wild adults | Hatchery adults (stray) | Percent wild |  |  |
| --- | --- | --- | --- | --- | --- |
| 1977 | 136 | No Data | 100.0% |  |  |
| 1978 | 417 | No Data | 100.0% |  |  |
| 1979 | 378 | 16 | 95.9% |  |  |
| 1980 | 311 | 42 | 88.1% |  |  |
| 1981 | 397 | 46 | 89.6% |  |  |
| 1982 | 569 | 39 | 93.6% |  |  |
| 1983 | 255 | 35 | 87.9% |  |  |
| 1984 | 431 | 129 | 77.0% |  |  |
| 1985 | 577 | 89 | 86.6% |  |  |
| 1986 | 373 | 56 | 86.9% |  |  |
| 1987 | 822 | 692 | 54.3% |  |  |
| 1988 | 522 | 699 | 42.8% |  |  |
| 1989 | 385 | 204 | 65.4% |  |  |
| 1990 | 339 | 182 | 65.1% |  |  |
| 1991 | 165 | 129 | 56.1% |  |  |
| 1992 | 280 | 403 | 41.0% |  |  |
| 1993 | 79 | 115 | 40.7% |  |  |
| 1994 | 135 | 147 | 47.9% |  |  |
| 1995 | 95 | 106 | 47.3% |  |  |
| 1996 | 85 | 168 | 33.6% |  |  |
| 1997 | 243 | 349 | 41.0% |  |  |
| 1998 | 214 | 380 | 36.0% |  |  |
| 1999 | 98 | 80 | 55.1% |  |  |
| 2000 | 325 | 421 | 43.6% |  |  |
| 2001 | 509 | 319 | 61.5% |  |  |
| 2002 | 734 | 988 | 42.6% |  |  |
| 2003 | 880 | 578 | 60.4% |  |  |
| 2004 | 282 | 177 | 61.4% |  |  |
| 2005 | 327 | 60 | 84.5% |  |  |
| 2006 | 256 | 86 | 74.9% |  |  |
| 2007 | 398 | 278 | 58.9% |  |  |
| 2008 | 305 | 105 | 74.4% |  |  |
| 2009 | 128 | 57 | 69.2% |  |  |
| 2010 | 621 | 265 | 70.1% |  |  |
| Average | 347 | 233 | 65.7% |  |  |

Shitike Creek - A picket weir has been installed seasonally, in various configurations, at Rkm 3.2 on Shitike Creek since 2001 (Figure 2).  The weir has been operated for purposes other than escapement estimates of adult spring Chinook salmon and steelhead.  From 2001 to 2004, the goal was enumeration of bull trout Salvelinus confluentus.  From 2005 to 2008, the goal was collection of post- spawned steelhead kelts for the Columbia River Inter-Tribal Fish Commission (CRITFC) kelt reconditioning project.  From 2004 to 2008, the weir was switched to video-only in early to mid June when daily maximum water temperatures reach 17° C as required by the operational permit.  In 2009 and 2010, the weir was installed in late May and was used to conduct video counts only.

From mid-March through early May, high water from rain-on-snow flood events sometimes overtops the weir in Shitike Creek, which creates bedload movement and scour holes around the pickets, (Figure 3), making enumeration of fish difficult.  Data on fish passage at the weir (Table 4) are therefore not reliable estimates of total escapement.  Video counts conducted at the Shitike Creek weir from 2004 to present (Table 5), provide a reasonably accurate estimate of the number of spring Chinook salmon passing upstream, if only because migration timing is during the summer months (May – Sept.) when low and steady flows allow better maintenance of the weir.

From 2000 through 2007, the CTWSRO and the USFWS implemented an out-planting project with the goal of increasing natural production of spring Chinook salmon in Shitike Creek (Table 6).  Adult hatchery spring Chinook in excess of broodstock needs were collected at the WSNFH and distributed to locations of suitable spawning habitat within Shitike Creek.  The majority of the fish were out-planted during August and September, when the fish were ripe and ready to spawn.  Table 7 presents data on the total adult spring Chinook salmon escapement into Shitike Creek from 2004 through 2009, combining net upstream video counts with out-plant data.  These are the only years in which we feel we have a relatively accurate estimate of escapement.

Redd Counts

Beginning in 1982 for steelhead, and 1986 for spring Chinook salmon, the CTWSRO conducted spawning ground surveys in the Warm Springs River watershed and Shitike Creek.  For both summer steelhead and spring Chinook salmon, two or three passes were performed on a subsample of stream reaches, or “index reaches.”  These index reaches tend to be areas with the greatest concentration of suitable spawning habitat, and comprise between 49 and 60% of the total stream mileage available to anadromous fish (Table 8.)

Figure 2.  Shitike Creek picket weir at typical winter flow conditions.

Table 4.  Net (upstream – downstream) adult fish handled and passed upstream at the Shitike Creek Weir, 2001 – 2008.  The weir was switched to video count when water temperatures reached 17o C (typically late May – mid June).  Note that weir was operated video-only beginning in late May starting in 2009.

| Year | Spring Chinook (wild) | Spring Chinook (marked) | Steelhead (wild) | Steelhead (marked) | Rainbow trout |
| --- | --- | --- | --- | --- | --- |
| 2001 | 73 | 3 | 74 | 52 | 791 |
| 2002 | 0 | 0 | 51 | 38 | 30 |
| 2003 | 41 | 0 | 11 | 24 | 153 |
| 2004 | 18 | -3 | 17 | 0 | 0 |
| 2005 | 1 | 0 | 43 | 14 | 199 |
| 2006 | 1 | 0 | 16 | 4 | 149 |
| 2007 | 0 | 0 | 29 | 61 | -43 |
| 2008 | 0 | 0 | 7 | 34 | -4 |

Figure 3.  Shitike Creek picket weir during high water event.

Table 5.  Net (upstream – downstream) upstream video count of adult fish 2004 – 2010.

| Year | Spring Chinook (wild) | Spring Chinook (marked) | Steelhead (wild) | Steelhead (marked) |
| --- | --- | --- | --- | --- |
| 2004 | 83 | 6 | 0 | 0 |
| 2005 | 30 | 4 | -4 | 0 |
| 2006 | 92 | 8 | 0 | 0 |
| 2007 | 61 | 19 | 0 | 0 |
| 2008 | 79 | 7 | 0 | 1 |
| 2009 | 44 | 24 | 1 | -2 |
| 2010 | 119 | 31 | -1 | 2 |

Table 6.  Summary of out-planted adult spring Chinook salmon in Shitike Creek, Brood

Years 2000 – 2007.

| Year |  | Out-planted Fish |  |
| --- | --- | --- | --- |
|  | Females | Males | Total |
| 2000 | 110 | 49 | 159 |
| 2001 | 123 | 76 | 199 |
| 2002 | 19 | 64 | 83 |
| 2003 | 155 | 110 | 265 |
| 2004 | 95 | 76 | 171 |
| 2005 | 103 | 75 | 178 |
| 2006 | 43 | 48 | 91 |
| 2007 | 5 | 7 | 12 |

Table 7.  Total spring Chinook salmon adult escapement into Shitike Creek, 2004 – 2010.

| Year | Net upstream video (wild) | Net upstream video (marked) | Out-planted | Total |
| --- | --- | --- | --- | --- |
| 2004 | 83 | 6 | 171 | 260 |
| 2005 | 30 | 4 | 178 | 212 |
| 2006 | 92 | 8 | 91 | 219 |
| 2007 | 61 | 19 | 12 | 92 |
| 2008 | 79 | 7 | 0 | 86 |
| 2009 | 44 | 24 | 0 | 68 |
| 2010 | 119 | 31 | 0 | 150 |

Table 8.  Spawning ground index reaches in Reservation streams.

| Stream | Number of index reaches | Rkm surveyed in index reaches | Rkm Total | % surveyed |
| --- | --- | --- | --- | --- |
| Beaver Creek* | 5 | 18.1 | 33.6 | 54% |
| Mill Creek* | 3 | 10.5 | 17.5 | 60% |
| Warm Springs River | 4 | 33 | 67.6 | 49% |
| Shitike Creek | 6 | 23.8 | 41.3 | 58% |

*Beaver Creek and Mill Creek are tributaries to the Warm Springs River.

Warm Springs River – An average of 289 Spring Chinook salmon redds have been found in the Warm Springs River Basin annually since 1986, with counts ranging from a high of 752 in 2000 to a low of 65 in 1995 (Figure 4).  In most years, the greatest abundance of redds (nearly 60%) were observed in the Schoolie to McKinley-Arthur reaches, between Rkm 42 and Rkm 60.  Counts have been below the 24-year average each year since 2005.  Fish per redd ratios (based on the number of fish passed above WSNFH) have ranged from 7.5 in 2005 to a low of 2.2 in 1998.

Steelhead redd counts in the Warm Springs River watershed have been as high as 97 in 2007 and as low as eight in 1996 (Figure 5).  However, spawning ground surveys for summer steelhead have been inconsistent in their implementation over the past 28 years in the Warm Springs River.  Prior to 2006, data exists for only 1982, 1988 and 1989.

Shitike Creek - An average of 29 spring Chinook salmon redds have been found in Shitike Creek since 1986, ranging from a high of 109 in 2004 to a low of 11 in 1999 (Figure 6).  The total number and location of spring Chinook salmon redds in Shitike Creek has changed significantly since implementation of the out-planting program (2000 – 2007).  From 1986 to 1999 the number of redds averaged 17.  The average had increased to 60 redds since 2000, but has declined since 2005.  The most recent numbers of redds observed (2008 – present) in Shitike Creek may be a fairly accurate representation of production without supplementation, since the out-planting of adults has not taken place since 2007.

The 33 observed summer steelhead redds in Shitike Creek in 2009 represents a decrease from the average of 46 redds observed between 1982 – 2009 (Figure 7).  Numbers of redds have generally increased in Shitike Creek since 1998, which may be attributed to the listing of the species and subsequent modifications to harvest regulations.  The highest number of redds observed in Shitike Creek was 137 summer steelhead redds in 2003.

Juvenile Rearing

Efforts to estimate numbers of rearing juvenile salmonids began on Reservation streams in the early 2000's with studies conducted on Shitike Creek (Dambacher 2002; Engle and Olson 2006).  The main objective of these studies was to estimate abundance of salmonids in Shitike Creek (Table 9).  Additional objectives of Engle and Olsen (2006) were to 1) identify micro-habitat selection (depth, water velocity, species association, cover use, temperature) by juvenile Chinook salmon, juvenile O. mykiss, and bull trout within Shitike Creek at varying densities, and 2) determine the relationship between micro-habitat selection and fish density in slow- and fast-water habitat units.

From 2006 through 2010 the CTWSRO fisheries production program implemented a new strategy for juvenile salmonid snorkel surveys.  The goal of these sampling efforts was to quantify the relative differences of juvenile Chinook salmon densities and counts among stream reaches in Shitike Creek, the Warm Springs River, Beaver Creek, and Mill Creek.

Figure 4.  Spring Chinook salmon redd counts in the Warm Springs River watershed above WSNFH, 1986 – 2010.

Figure 5.  Steelhead redd counts in the Warm Springs River watershed 1982 – 2010.  No data were collected in 1983, 1984, and 1993 due to high water.  Surveys were not conducted in the Warm Spring River in 1983 – 1997 and 2000 – 2005.

Figure 6.  Spring Chinook salmon redd counts in Shitike Creek 1986 – 2010.  Outplanting project occurred from 2000 through 2007.

Figure 7.  Steelhead redd counts in Shitike Creek, 1982 – 2010.

Table 9.  Slow-water populations estimates in sampled reaches of Shitike Creek, 2001 – 2004.  Confidence intervals (95%) on either side of the estimate are given in parentheses.  From Engle and Olsen (2006).

|  | Chinook salmon |  |  |  |  | O. mykiss |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reach | 2001 | 2003 | 2004 | 2005 |  | 2001 | 2003 | 2004 | 2005 |
| 1 | 10,991 (3,322) | 2,435 (908) | 4,732 (2,611) | 5,554 (1,655) |  | 2,703 (946) | 2,240 (711) | 2,237 (2,611) | 3,663 (1,655) |
| 2 | 168 (0) | -- | 179 (553) | -- |  | 121 (0) | -- | 593 (553) | -- |
| 3 | 2,968 (1,474) | 2,075 (649) | 3,958 (1,973) | 3,485 (1,276) |  | 835 (366) | 1,474 (475) | 2,120 (1,973) | 1,403 (1,276) |
| 5 | 1,158 (1,362) | 66 (53) | 2,462 (1,368) | 3,363 (734) |  | 0 | 180 (87) | 161 (1,368) | 111 (734) |
| Total | 15,285 (3,897) | 4,576 (1,118) | 11,332 (1,778) | 12,402 (2,215) |  | 3,659 (1,014) | 15,285 (3,897) | 3,894 (859) | 4,576 (1,118) |

The CTWSRO snorkeling strategy differed from past efforts in that the focus was not so much on microhabitat (where is a fish within a pool?) or habitat unit usage (are there more fish in pools or riffles?), but rather on reach and stream level differences (are there higher densities of fish in Mill Creek or Beaver Creek?).  The primary units of analysis were pools within streams and stream reaches.  The estimated densities of juvenile fish in randomly selected pools within streams reaches served as sample points within each unit.

For all three studies listed above, a three pass, bounded count methodology (Dambacher 2002) was used to generate population estimates for each pool.  The bounded count estimator was:

Results indicate that densities of rearing juvenile Chinook salmon vary across years and streams (Table 10).  The lowest densities of juvenile Chinook salmon were found in the Warm Springs River in three of the four years between 2006 and 2009.  Shitike Creek had the lowest density in 2007, but had the highest density in 2008.  Mill Creek had the highest density in three of the four years, and the highest average density observed.

Densities of juvenile O. mykiss were slightly lower than those of juvenile Chinook salmon overall, but appear to be more stable (Table 11).  Similar to estimates for juvenile Chinook salmon, the highest estimated densities were observed in Mill Creek in 2007.  The lowest densities of O. mykiss were observed in the Warm Springs River in all years.

Table 10.  Average non-zero densities of juvenile Chinook salmon (fish / 100 m2), and percentage of snorkel units with zero counts in streams on the Warm Springs Reservation 2006 – 2009.  2010 data has yet to be analyzed.

| Stream | __2006__ |  | __2007__ |  | __2008__ |  | __2009__ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average Density | % zero counts | Average Density | % zero counts | Average Density | % zero counts | Average Density | % zero counts |
| Shitike Cr. | 2.88 | 66% | 10.78 | 24% | 17.00 | 18% | 19.22 | 41% |
| Mill Cr. | 11.68 | 40% | 34.78 | 50% | 13.08 | 36% | 22.42 | 25% |
| Beaver Cr. | 2.17 | 76% | 26.08 | 38% | 3.55 | 68% | 12.95 | 54% |
| Warm Sprgs R. | 0.40 | 66% | 14.36 | 20% | 1.33 | 70% | 2.55 | 73% |

Table 11.  Average non-zero densities of juvenile O. mykiss (fish / 100 m2), and percentage of snorkel units with zero counts in streams on the Warm Springs Reservation 2006 – 2009.  Includes all O. mykiss observed under 200mm.  2010 data has yet to be analyzed.

| Stream | __2006__ |  | __2007__ |  | __2008__ |  | __2009__ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average Density | % zero counts | Average Density | % zero counts | Average Density | % zero counts | Average Density | % zero counts |
| Shitike Cr. | 19.38 | 21% | 13.35 | 24% | 12.71 | 18% | 11.91 | 38% |
| Mill Cr. | 16.34 | 0% | 22.63 | 0% | 6.50 | 0% | 9.28 | 13% |
| Beaver Cr. | 5.64 | 45% | 11.64 | 13% | 6.72 | 68% | 14.49 | 38% |
| Warm Sprgs R. | 1.99 | 33% | 2.31 | 20% | 1.53 | 40% | 1.91 | 64% |

During snorkel surveys, several of the sampled pools were found to have no juvenile Chinook salmon (range 18% and 76%) or juvenile O. mykiss (range 0% - 68%).  A greater percentage of empty pools occur in years with overall lower density estimates.  Densities of both species vary more among years than among streams within a year.

Juvenile Outmigration

Warm Springs River – An outmigrant trap has been operated near the mouth of the Warm Springs River since 1976 to provide data on migration timing and abundance of migrating wild juvenile spring Chinook salmon and steelhead.  The original Humphreys-type scoop trap was operated from 1976 until 2002, at which time it was replaced by the current 8-foot rotary screw trap.  The current screw trap is located at Rkm 1.3.  In 2010, the trap was operated from March 9 to June 5 and October 6 to December 3, which are typical dates of annual sampling periods.

The estimated number of outmigrating juvenile spring Chinook salmon from the Warm Springs River averages about 86,000 per year, with estimates ranging as high as 211,529 for brood year 2004 and as low as 3,784 for brood year 1995 (Table 12).  An average of 40% of the outmigrants are age-1 spring smolts (range 16% - 80%), 53% are age-0 fall migrants (range19% - 73%), and 7% are age-0 spring migrants (range 0% - 29%).

Since 2005, an average of 321 juvenile O. mykiss has been captured annually during the spring trapping period at the Warm Springs River outmigrant trap.  Of these, the largest proportion (49%) are in the 50-100 mm range, followed by an average of 33% in the 100-150 mm range, 16% in the 150-200mm range, and 3% below 50mm.  Additionally, an average of 11% of these fish has been qualitatively identified as steelhead smolts (silvery and laterally compressed).  However, we currently have no definitive method of determining whether an O. mykiss juvenile is destined for anadromy or residency.  In the fall trapping period, an average of 59 juvenile O. mykiss are captured annually, but the sizes of fish are more evenly spread, with an average of 40% of the fish below 50mm, 33% in the 50-100 mm range, 15% between 100-150 mm, and 12% above 150mm.

Shitike Creek - A 5-foot rotary screw trap has been operated near the mouth of Shitike Creek during the spring and fall since 1995.  The trap is located just upstream of the footbridge at the Warm Springs Forest Products compound at Rkm 0.5.  In 2010, the trap was operated from March 5 to June 20 and October 16 to December 4, which represents typical dates of operation.

The estimated number of outmigrating juvenile spring Chinook salmon from Shitike Creek averaged only about 125 outmigrants between brood years 1995 and 1999, with a low of only 3 individuals in brood year 1995 (Table 13).  These low production numbers were one of the reasons the CTWS decided to begin a project of outplanting adult hatchery salmon into the creek in 2000 (described above, see Table 6).  Since 2000, Shitike Creek has seen an average of approximately 13,990 outmigrants per year, with estimates ranging as high as 43,276 for brood year 2004 and as low as 1,590 for brood year 2005 (Table 13).  An average of 22 % of the outmigrants are age-1 spring smolts (range 9% - 33%), 64% are age-0 fall migrants (range 29% - 89%), and 14% are age-0 spring migrants (range 0% - 55%).

Table 12.  Outmigration estimates of age-0 spring, age-0 fall, and age-1 spring (smolt)

juvenile spring Chinook salmon from the Warm Springs River, brood years 1977 – 2009.

| Brood year | Age 0+ spring | Age 0+ fall | Age 1+ spring | Total outmigration estimate |
| --- | --- | --- | --- | --- |
| 1975 | no data | 25,795 | 43,250 | 69,045 |
| 1976 | no data | 47,041 | 26,043 | 73,084 |
| 1977 | no data | 25,125 | 45,204 | 70,329 |
| 1978 | no data | 74,727 | 57,216 | 131,943 |
| 1979 | no data | 24,930 | 25,628 | 50,558 |
| 1980 | no data | 20,579 | 14,656 | 35,235 |
| 1981 | no data | 29,238 | 14,647 | 43,885 |
| 1982 | no data | 67,688 | 30,619 | 98,307 |
| 1983 | 1,548 | 89,469 | 31,081 | 122,098 |
| 1984 | 605 | 61,986 | 34,842 | 97,433 |
| 1985 | 1,357 | 35,974 | 38,335 | 75,666 |
| 1986 | 9,305 | 47,130 | 35,649 | 92,084 |
| 1987 | 7,303 | 59,172 | 27,514 | 93,989 |
| 1988 | 6,635 | 56,033 | 40,336 | 103,004 |
| 1989 | 6,100 | 42,697 | 33,146 | 81,943 |
| 1990 | 5,430 | 51,365 | 46,929 | 103,724 |
| 1991 | 5,500 | 14,709 | 14,307 | 34,516 |
| 1992 | 2,481 | 26,098 | 29,313 | 57,892 |
| 1993 | 5,607 | 14,197 | 13,663 | 33,467 |
| 1994 | 758 | 51,057 | 7,689 | 59,504 |
| 1995 | 54 | 2,184 | 1,546 | 3,784 |
| 1996 | 6,335 | 78,468 | 70,423 | 155,226 |
| 1997 | 24,649 | 56,065 | 24,528 | 105,242 |
| 1998 | 258 | 29,600 | 9,121 | 38,979 |
| 1999 | 3,077 | 19,151 | 76,318 | 98,546 |
| 2000 | 15,916 | 0 | 64,731 | 80,647 |
| 2001 | 4,076 | 45,234 | 73,493 | 122,803 |
| 2002 | 5,908 | 38,898 | 49,961 | 94,767 |
| 2003 | 6,506 | 31,108 | 16,940 | 54,554 |
| 2004 | 61,270 | 117,247 | 33,012 | 211,529 |
| 2005 | 351 | 43,841 | 15,787 | 59,979 |
| 2006 | 11,236 | 84,187 | 36,897 | 132,320 |
| 2007 | 21 | 32,596 | 16,408 | 49,025 |
| 2008 | 718 | 64,022 | TBD | TBD |
| 2009 | TBD | TBD | TBD | TBD |

Table 13.  Outmigration estimates of age-0 spring, age-0 fall, and age-1 spring (smolt)

juvenile spring Chinook salmon from Shitike Creek, brood years 1994 – 2009.

| Brood year | Age 0+ spring | Age 0+ fall | Age 1+ spring | Total outmigration estimate |
| --- | --- | --- | --- | --- |
| 1994 |  | 44 | 3 | 47 |
| 1995 |  | 3 | 0 | 3 |
| 1996 |  | 27 | 8 | 35 |
| 1997 | 159 | 33 | 55 | 247 |
| 1998 | 2 | 136 | 82 | 220 |
| 1999 | 0 | 5 | 190 | 195 |
| 2000 | 6,565 | 4,204 | 3,859 | 14,628 |
| 2001 | 1,736 | 4,845 | 3,277 | 9,858 |
| 2002 | 1,041 | 7,457 | 1,403 | 9,901 |
| 2003 | 3,560 | 11,165 | 6,337 | 21,062 |
| 2004 | 8,339 | 31,222 | 3,715 | 43,276 |
| 2005 | 0 | 1,418 | 172 | 1,590 |
| 2006 | 196 | 6,124 | 2,808 | 9,128 |
| 2007 | 0 | 1,992 | 492 | 1,992 |
| 2008 | 201 | 3,335 | TBD | TBD |
| 2009 | TBD | TBD | TBD | TBD |

Since 2005, an average of 4,022 juvenile O. mykiss have been captured annually at the outmigrant trap on Shitike Creek, which is more than 10 times the number captured annually in the Warm Springs River during the same time period.  The seasonal proportion of these fish outmigrating from the creek has varied over this time period.  In 2005 and 2006, more juvenile O. mykiss were captured in the fall (77% and 83%, respectively) than in the spring.  Between 2007 and 2010 that number was reversed, with more fish (between 67% and 85%) captured in the spring.  Fish between 50-100mm are the most abundant in both the spring and fall, representing 89% and 62% of the fish captured, respectively.  Fish smaller than 50mm represent 33% of the catch in the fall, but only 4% in the spring.  Fish over 100 mm represent 11% of the catch in the spring, and about 5% of the catch in the fall.

Spring Chinook Salmon Smolt to Adult Relationships

Warm Springs River - The number of adult spring Chinook salmon passing upstream of the WSNFH and the subsequent number of outmigrants are positively correlated (r2 = 0.37, p< 0.001, Figure 8), and recruits per spawner and the number of spawners are negatively correlated (r2 = 0.14, p = 0.035, Figure 9), indicating a potential density-dependent negative effect on survival.  On average, each spawner produces about 73 outmigrants, with a range of 16 to 200 (Figure 10).  Smolt-to-adult return (SAR) rates have averaged 3.5% from 1977 to 2007, ranging from 0.9% to 31.7% (Table 14).

Figure 8.  Relationship between adult spring Chinook salmon returns upstream of the WSNFH and estimated number of outmigrants, brood years 1977 – 2007.

Figure 9.  Relationship between adult spring Chinook salmon returns upstream of the WSNFH and outmigrants per spawner, brood years 1977 – 2007.

Figure 10.  Wild adult returns and outmigrants per spawner in the Warm Springs River, brood years 1977 – 2007.

Relationships between SARs and physical (e.g., ocean conditions) or biological (e.g., smolt density) factors are not readily apparent.  SARs exceeded 10% for the 1981, 1995, and 1998 brood years.  As estimated by the Pacific Decadal Oscillation (PDO) index, fish from the 1981 brood experienced relatively warm ocean conditions in all years, fish from 1995 experienced mostly warm conditions, and fish from 1998 experienced cool conditions.  SARs were less than 2% for brood years 1989 – 1991, when fish experienced a mix of warm and cool ocean conditions.  Fish from the other two brood years with SARs below 2% (2001 and 2004) also experienced a mix of warm and cool conditions.  Some of the highest SARs resulted from low numbers of smolts; however, the high number of smolts from the 1996 brood experienced mostly warm ocean conditions, yet had an above average SAR.

Shitike Creek - Data on smolt-to-adult return ratios for spring Chinook salmon in Shitike Creek are limited, as escapement numbers for adult fish exist only from the years 2004 – present, and no data exist on the age composition of the adult run.  Table 15 presents SAR estimates using the assumption that the all adult fish returns are 4 year old.

Table 14.  Warm Springs River wild spring Chinook salmon outmigration estimates and adult returns back to the Deschutes River brood years 1975 – 2007.  Return estimates are returns to the WSNFH plus estimated harvest in the Deschutes River.

| Brood year | Total outmigrant estimate | Smolt outmigrant estimate | Total adult return* | Outmigrant to adult return | Smolt to adult return |
| --- | --- | --- | --- | --- | --- |
| 1975 | 69,045 | 43,250 | 1,891 | 2.7% | 4.4% |
| 1976 | 73,084 | 26,043 | 1,547 | 2.1% | 5.9% |
| 1977 | 70,329 | 45,204 | 1,691 | 2.4% | 3.7% |
| 1978 | 131,943 | 57,216 | 2,009 | 1.5% | 3.5% |
| 1979 | 50,558 | 25,628 | 2,077 | 4.1% | 8.1% |
| 1980 | 35,235 | 14,656 | 1,162 | 3.3% | 7.9% |
| 1981 | 43,885 | 14,647 | 1,807 | 4.1% | 12.3% |
| 1982 | 98,307 | 30,619 | 2,770 | 2.8% | 9.0% |
| 1983 | 122,098 | 31,081 | 2,743 | 2.2% | 8.8% |
| 1984 | 97,433 | 34,842 | 2,344 | 2.4% | 6.7% |
| 1985 | 75,666 | 38,335 | 2,274 | 3.0% | 5.9% |
| 1986 | 92,084 | 35,649 | 2,938 | 3.2% | 8.2% |
| 1987 | 93,989 | 27,514 | 1,372 | 1.5% | 5.0% |
| 1988 | 103,004 | 40,336 | 1,830 | 1.8% | 4.5% |
| 1989 | 81,943 | 33,146 | 564 | 0.7% | 1.7% |
| 1990 | 103,724 | 46,929 | 453 | 0.4% | 1.0% |
| 1991 | 34,516 | 14,307 | 128 | 0.4% | 0.9% |
| 1992 | 57,892 | 29,313 | 1,487 | 2.6% | 5.1% |
| 1993 | 33,467 | 13,663 | 898 | 2.7% | 6.6% |
| 1994 | 59,504 | 7,689 | 275 | 0.5% | 3.6% |
| 1995 | 3,784 | 1,546 | 490 | 12.9% | 31.7% |
| 1996 | 155,226 | 70,423 | 3,520 | 2.3% | 5.0% |
| 1997 | 105,242 | 24,528 | 2,169 | 2.1% | 8.8% |
| 1998 | 38,979 | 9,121 | 1,557 | 4.0% | 17.1% |
| 1999 | 98,546 | 76,318 | 1,670 | 1.7% | 2.2% |
| 2000 | 80,647 | 64,731 | 2,363 | 2.9% | 3.7% |
| 2001 | 122,803 | 73,493 | 968 | 0.8% | 1.3% |
| 2002 | 94,767 | 49,961 | 1,018 | 1.1% | 2.0% |
| 2003 | 54,554 | 16,940 | 489 | 0.9% | 2.9% |
| 2004 | 211,529 | 33,012 | 578 | 0.3% | 1.8% |
| 2005 | 59,979 | 15,787 | - | - | - |
| 2006 | 132,320 | 36,897 | - | - | - |
| 2007 | 49,025 | 16,408 | - | - | - |
| Average | 82,882 | 33,310 | 1,569 | 2.5% | 6.3% |

* Adult return number includes 3 -, 4 -, and 5 – year olds from that brood year, based upon age-at-return run reconstruction.

Table 15.  Smolt-to-adult and outmigrant-to adult return ratio estimates for Shitike Creek Spring Chinook salmon, brood years 2001 – 2007.

C. Rationale and significance to regional programs

Northwest Power and Conservation Council Fish and Wildlife Program

The NPCC’s Columbia River Basin Fish and Wildlife Program (Program; NPCC 2009) is intended to integrate ESA requirements, Northwest Power Act requirements, and the policies of the States and Indian Tribes of the Columbia River Basin into a comprehensive program grounded in a solid scientific foundation.  The NPCC recognizes that the Tribes have vital interests directly affected by activities covered in the Program and that the United States has a trust obligation to preserve and protect the natural resources reserved by or protected in treaties, executive orders, and federal statutes.  The NPCC also recognizes that significant interaction and cooperation with the Tribes as co-managers of affected fish and wildlife resources will be necessary to fully implement the Program and its goals, objectives, and strategies.

The proposed project aligns with several elements of the Monitoring, Evaluation, Research, and Reporting Strategies in the 2009 Program, including contributing to tracking the status and trends of priority focal species; determining the effectiveness of projects implemented by the Program; clear linkage to subbasin goals, limiting factors, priority reaches, and focal species; monitoring representatives of entire fish populations; and methods and protocols consistent with guidelines.  Key indicator naturally spawning populations will be monitored to provide detailed stock status information.  In addition, this project will provide for the collection of population status, life history, and other data of wild and naturally spawning populations.

Deschutes Subbasin Management Plan

The Deschutes Subbasin Plan identified several areas of monitoring needs for the subbasin, including research and evaluation strategies for each of a number of sub-watersheds.  The proposed project directly addresses research and evaluation strategies for the Warm Springs River and Shitike Creek sub-watersheds, including:

- Determine life history characteristics of redband trout, summer steelhead, bull trout and Pacific lamprey in the Warm Springs River habitat complex.

- Continue annual steelhead, bull trout and spring Chinook spawning surveys in the Warm Springs River habitat complex.

- Determine life history characteristics of focal fish species in the Shitike Creek habitat complex.

- Evaluate the success of adult spring Chinook supplementation on increasing natural production in the Shitike Creek habitat complex.

- Continue annual bull trout, steelhead and spring Chinook spawning surveys in the Shitike Creek habitat complex.

Middle Columbia River Steelhead Recovery Plan

Within the recovery plan for Middle Columbia River steelhead, populations in the Deschutes River Subbasin are thoroughly covered in the appendix titled Conservation and Recovery Plan for Oregon Steelhead Populations in the Middle Columbia River Steelhead Distinct Population Segment http://www.nwr.noaa.gov/Salmon-Recovery-Planning/Recovery-Domains/Interior-Columbia/Mid-Columbia/upload/Mid-C-Oregon.pdf.  The research, monitoring, and evaluation section of the recovery plan for Oregon steelhead populations in the Middle Columbia River DPS is based partly on principles and concepts from the NMFS guidance document, Adaptive Management for ESA-Listed Salmon and Steelhead Recovery: Decision Framework and Monitoring Guidance (May 1, 2007) http://www.nwr.noaa.gov/Salmon-Recovery-Planning/ESA-Recovery-Plans/upload/Adaptive_Mngmnt.pdf.  The recovery plan also follows the principles of adaptive management, and acknowledges that several types of monitoring are needed to support adaptive management, including status and trend monitoring, action effectiveness monitoring, implementation and compliance monitoring, and uncertainties research.

This proposal addresses objectives from the research, monitoring, and evaluation section of the recovery plan, including:

- Determine the status and trend in the abundance of natural-origin spawners and compare to viability abundance criteria for the Deschutes River populations.

- Determine the status of the spatial structure of each population based on current and historically utilized habitat.

- Determine the freshwater productivity of each population relative to viability criteria and identify primary habitat factors limiting freshwater production.

- Determine the effect of habitat degradation and habitat restoration activities on the abundance, productivity, and distribution of the Deschutes River populations.

2008 FCRPS Biological Opinion

The FCRPS Biological Opinion calls for the development and implementation of a basin-wide hierarchical monitoring program.  The monitoring program outlined in this proposal will provide statistically rigorous estimates of the status, trends, and distribution of adult and juvenile salmon and their habitats.  This proposed project addresses several action items under the 2008 FCRPS Biological Opinion and the FCRPS Adaptive Management Implementation Plan.  Under Research, Monitoring, and Evaluation Actions this proposal addresses several strategies and alternatives including:

RPA 50 (Fish Population Status Monitoring) —This RPA is fundamentally supported by the proposed monitoring program.  This monitoring program contains all of the essential elements of the NOAA Fisheries framework for monitoring.  We provide PIT tagged steelhead and Chinook for the evaluation of the PIT-tag detection system in the Columbia River basin and tributaries.

RPA 52 (Monitor and Evaluate Fish Performance within the FCRPS) and RPA 54 (Monitor and Evaluate Effects of Configuration and Operation Actions) — We will provide about 4,000 PIT-tagged Chinook salmon and about 1,000 PIT-tagged steelhead smolts/year that can be  used for the evaluation of survival at FCRPS facilities for both juveniles and adults.

RPA 66 & 67 (Monitor and Evaluate the Caspian Tern & Double-Crested Cormorant Populations in the Columbia River Estuary) — An essential component of this monitoring has been the recovery of PIT tags in rookeries to quantify and identify the composition of the forage utilized by these avian piscivores.  We will provide about 4,000 PIT-tagged Chinook salmon and about 1,000 PIT-tagged steelhead smolts/year that can be incorporated into this monitoring.

Columbia Basin Fish Accord

In 2008 the “Federal Action Agencies” (Bonneville Power Administration (BPA), the U.S. Army Corps of Engineers (USACE), and the U.S. Bureau of Reclamation (USBR) and the Confederated Tribes of the Warm Springs Reservation of Oregon, the Confederated Tribes of the Umatilla Indian Reservation, the Confederated Tribes and Bands of the Yakama Nation, and the Columbia River Inter-Tribal Fish Commission signed the Columbia Basin Fish Accords Memorandum of Agreement (MOA) between the Tribes and FCRPS Action Agencies (The Accord).  The Accord addresses direct and indirect effects of construction, inundation, operation and maintenance of the Federal Columbia River Power System (FCRPS) and the USBRs Upper Snake River Projects, on fish resources of the Columbia River Basin.  The Accord includes requirements that funded habitat actions are linked to biological benefits based on limiting factors for ESA-listed fish, and that actions be consistent with recovery plans and subbasin plans.  The proposed project, originally planned as two separate projects (one for steelhead and one for Chinook salmon) is specifically included as a project to be funded as part of The Accord.

Anadromous Salmonid Monitoring Strategy

In 2009, federal, state, and tribal fish and wildlife managers worked collaboratively through a series of sub-regional and regional workshops and developed a coordinated Anadromous Salmonid Monitoring Strategy (ASMS).  The focus of the ASMS is to meet monitoring and adaptive management need of the NPCC’s Program, ESA recovery plans, the FCRPS BiOp, and federal, state, and tribal fish and wildlife programs in a cost-effective manner.  Although still draft, the ASMS includes guidelines for viable salmonid population monitoring, rationale for guiding monitoring, and specific strategies for prioritizing funding for middle Columbia River steelhead.  A specific strategy addressed by the proposed project is the focusing of smolt monitoring in Bakeoven Creek, Warm Springs River, Shitike Creek, Buckhollow Creek, and Trout Creek.

D. RELATIONSHIP TO OTHER CTWSRO PROJECTS

Table 16. Relationship to other projects

| Funding Source | Project # | Project Title | Brief Relationship |
| --- | --- | --- | --- |
| USFWS | 13310-7-J141 | Cooperative Agreement between USFWS and CTWSRO | Cooperatively manage the Warm Springs National Fish Hatchery, investigate effects of hatchery fish on wild fish populations in the Deschutes River Basin. |
| BPA | 2008-301-00 | CTWSRO Deschutes River Restoration Program | Develop habitat restoration activities directed at increasing salmonid production in Reservation streams; share personnel to monitor effectiveness of habitat restoration activities. |
| PCSRF | 2010 – 3 - 03 | Columbia River Stock Evaluation | Share Personnel and equipment. Project is directed at monitoring and providing support to Tribal Spring Chinook ceremonial fishery on. Columbia River. |
| BPA | 2008-307-00 | CTWSRO Sockeye Development | Share personnel and equipment. Project is focused on monitoring kokanee population dynamics in Lake Billy Chinook and investigating methods of reestablishing sockeye runs in the Deschutes Basin. |
| BIA | N/A | CTWSRO Harvest Management | Manage tribal fisheries harvest both on- and off-reservation. Work with Tribal Council and committees to develop harvest seasons for tribal members and non-tribal members, perform creel surveys, provide presence and education to fishermen. |

PROJECT HISTORY

This is a newly proposed BPA Project, but it will continue monitoring efforts that have been ongoing in Reservation streams since the late 1970’s (See Table 1).

PROPOSAL BIOLOGICAL GOALS AND OBJECTIVES

The following goals include activities that will be implemented in 2011.  Effort may be expanded in future years to include more extensive or intensive monitoring as appropriate.  Justification will be provided prior to any expansion.

Goal 1: Continue annual life stage monitoring of wild spring Chinook salmon and steelhead and in the Warm Springs River and Shitike Creek.

Goal 2: Provide management and co-management direction of fisheries resources in the Deschutes River Basin.

Goal 1: Continue annual life stage monitoring of wild spring Chinook salmon and steelhead in the Warm Springs River watershed and Shitike Creek.

The primary goal of this project is to understand the status and trends of wild spring Chinook salmon and steelhead in CTWSRO streams.  This includes continuing some ongoing efforts, improving others, and implementing new efforts.  Efforts to be continued or improved include (1) monitoring adult escapement, (2) conducting spawning ground surveys (redd counts), (3) evaluating juvenile rearing, and (4) monitoring juvenile outmigration.  New efforts include PIT-tagging outmigrating juvenile spring Chinook salmon and steelhead.

All methods draw on established monitoring protocols for spring Chinook salmon and steelhead in the interior Columbia Basin, as described in the Monitoring Strategy for the Upper Columbia Basin (Hillman 2006), BPA’s ISEMP (Terraqua 2008; Moberg and Ward 2008; Murdoch and Nelle 2008; Nelle and Moberg 2008; Tussing 2009), and the Salmonid Field Protocols Handbook (Johnson et al. 2007).  Although many of these protocols were created for monitoring programs established in other subbasins (e.g. the Wenatchee, Entiat, John Day, and Salmon), the species in question and instream river conditions are similar to those in the Deschutes Subbasin and are therefore applicable to our program.

As described in Hillman (2006), status and trend monitoring programs are designed to answer two basic questions:

- What are the current habitat conditions and abundance, life-stage survival, and age-composition of fish species (status monitoring)?

- How do these factors change over time (trend monitoring)?

Creating a monitoring program to answer these questions will require the implementation of probabilistic sampling designs, standardized collection protocols, valid statistical design, and consistent data analysis and reporting.  It is our intention to use the wide range of literature available to us to create a spring Chinook salmon and steelhead monitoring program that is consistent with similar programs in the Interior Columbia Basin.

Adult Escapement

The number of adult fish reaching spawning grounds (escapement) is a critical component of fisheries management and stock assessment of salmonid populations (Zimmerman and Zabkar 2007).  Weirs are widely considered to provide the most accurate escapement data and are often used as the standard to which other escapement methods are compared (Parsons and Skalski 2009).  Quantification of adult spring Chinook salmon and steelhead escapement into CTWSRO streams will be accomplished through the use of a hatchery ladder weir at the WSNFH on the Warm Springs River (CTWSRO and USFWS 2007) and a resistance board weir (Stewart 2002, 2003) in Shitike Creek.

Warm Springs River

The barrier weir/fish ladder complex located at the WSNFH is operated by the USFWS.  The USFWS provides weekly counts of fish to the CTWSRO throughout the migration season of both spring Chinook salmon and steelhead (CTWSRO and USFWS 2007).

Shitike Creek

A picket weir has been operated at Rkm 1.0 in Shitike Creek since 2001.  From 2008 through 2010 the weir was installed in May (after spring high water events) in combination with a video camera and digital video recorder to estimate escapement of adult spring Chinook salmon.

Design and Installation - Operation of the picket weir during high water events in spring has complicated the estimation of steelhead escapement.  The need for a weir that can operate under both high and low water conditions and can be used for multiple species is desired by the CTWSRO and other fish managers in the Deschutes Basin.  CTWSRO intends to solicit a contract with an experienced consultant to develop a design and budget for the construction and installation of a weir in Shitike Creek that can meet these requirements in 2011, with construction and installation in 2012.

Field Methods - The weir will most likely be installed in the same location as the current picket weir, but the ultimate location will be developed in consultation with the chosen contractor.  The weir will be designed to operate year-round to capture fish traveling both up and downstream.  Since the run size of adult salmonids into Shitike Creek is relatively small (dozens to hundreds of fish each year), the weir will be operated to capture all migrating fish, and will be checked at a minimum of twice a day, with more frequent checks during the peak of the spawning migration.  The weir will be operated seven days a week during the steelhead migration season (February–June) and will be switched to video-monitoring only in late May to mid-June or when maximum daily water temperatures reach 17 °C (62°F), the maximum temperature allowed when handling steelhead, and will remain video-only throughout the summer and fall.  The weir will be reactivated to capture fish in the late winter, in time for the steelhead runs.

Biological information will be collected on all captured (handled) fish, including date of capture, sex, origin (hatchery or wild), length, weight, whether the fish is pre-or post- spawned (if determinable) and direction of migration (upstream or downstream).

Data Analysis - Weirs are designed to obtain an absolute count of migrating fish (Zimmerman and Zabkar 2009).  The total population of migrating steelhead will be obtained by summing daily counts of fish captured at the weir.  However, we will mark all adult steelhead captured in the upstream box with a PIT tag and an external mark (e.g. an opercle punch or a floy tag) before passing upstream as a method of determining weir efficiency.  Fish will be recaptured either as carcasses during spawning surveys, or as post-spawn migrants captured in the downstream box at the weir.  Surveyors will examine any carcasses observed during spawning surveys for marks and/or PIT tags.  Downstream migrating steelhead will be examined for marks and scanned for PIT tags, before release downstream.  If less than 100% of these recaptured fish (carcass + downstream migrants) are marked, we will utilize a mark/recapture methodology to estimate the total spawning escapement of steelhead.

Currently, all steelhead (regardless of wild- or hatchery-origin) pass into Shitike Creek.  The tribal policy regarding removal or passage of hatchery fish once this weir is constructed and installed has yet to be determined.

Estimating Chinook salmon spawning escapement will be conducted using video counting.  A camera box with a DVR will be placed in the weir that allows free upstream and downstream passage of migrating adult salmon.  Using the assumptions that all Chinook salmon that are observed passing upstream of the weir will either 1) spawn and die in Shitike Creek or 2) migrate back downstream of the weir without spawning, the total escapement of adult Chinook will be estimated as:

N(t) = N(up) - N(down)

Where:

N(t) = total population

N(up) = upstream migrating fish observed in video counts

N(down) = downstream migrating fish observed in video counts

Spawning Ground Surveys (Redd Counts)

Spawning ground surveys for spring Chinook salmon and summer steelhead will be conducted on reaches in Beaver, Mill, and Badger Creeks, as well as the upper and lower Warm Springs River in the Warm Springs watershed, and in Shitike Creek.  Spring Chinook salmon surveys will take place in late August to early October, and surveys for steelhead will occur between mid-April and June.

Reach Selection and Sampling Strategy

Our established index reaches encompass just over 53% of the total available habitat (Table 8), exceeding the 10% recommended by Gallagher et al (2007).  These index reaches were selected because they contain most prime spawning habitat and are accessible.  Each reach will be surveyed up to three times (no more than two weeks apart) over the course of a six or seven week period.

Our index reaches do not cover the entire range of spawning, primarily because of limited access to some of the canyon reaches.  Most of these non-index reaches have been surveyed on a random basis over the past two decades, and have either limited spawning habitat or have not been selected for spawning by either spring Chinook salmon or steelhead.

To facilitate statistically valid results, we will implement a rotating panel design to survey approximately 20% (15km) of the non-index reaches annually (for both spring Chinook salmon and steelhead) in a rotating panel design.  With this methodology, we will survey approximately 62.5% of available spawning habitat each year, and will sample all potential spawning reaches at least once between 2011 and 2015 (Table 17).  Random survey panels will be selected prior to beginning spawning surveys each season, with selected reaches surveyed once at the end of the respective spawning seasons (October for spring Chinook salmon; June for steelhead).

Field Methods

Survey techniques will be adapted from Gallagher et al. (2007).  A team of two or three surveyors will begin at the top of each reach and walk downstream.  Surveys on the Warm Springs River may be made in an inflatable kayak.  For each survey, the date, time, and beginning and ending water temperature will be recorded.  When a redd is identified, it will be counted and a flag will be placed on a nearby riparian tree.  The approximate length, width, depth (at the top of the redd) and location (left bank, right bank, mid-channel) will be recorded for each redd.  GPS locations will be noted at the beginning and end of each reach and at each individual redd.  GPS data will be downloaded into a SQL server database.

Sources of Error

Observation error may be made in one of three ways for redd surveys: 1) completely missing a redd for reasons such as water visibility, water depth, concealment by the growth of periphyton, or lack of training; 2) misidentifying a redd that was constructed by a different species; or 3) misidentifying and counting a redd that was either caused by stream hydraulics or by “test digs” in which fish disturb the substrate but do not actually deposit eggs.

The first source of error will be addressed by rigorous crew training and ensuring that each crew has at least one member experienced in redd surveys.  These crew members will serve as the “lead observer” for the crew, and will make the final decision as to whether a redd is real, a “test”, or created by hydraulics.  Surveys will not be conducted when water conditions impair the visibility of redds.  Conducting surveys no more than two weeks apart will allow for redd identification and counting when they are “fresh” and before the growth of periphyton can conceal them.

The chances of misidentifying a spring Chinook salmon redd as that of another species are minimal because of the large size of redds and because no other species spawn at the same time.  The likelihood of misidentifying steelhead redds is also small, although the tail-end of the steelhead spawning season coincides with the beginning of the rainbow (redband) trout spawning, particularly in the lower reaches of Shitike Creek.  Observations of fish on redds in Shitike Creek indicate that rainbow trout make smaller redds and use smaller substrate than do the larger steelhead (CTWSRO, unpublished data).  The judgment of whether a potential redd is a “test dig” or created by stream hydraulics will be made by the most experienced member of the field crew.

Data Analysis

Estimates of the total number of redds will be made by summing the number of redds counted in each of the index areas, and adding a “correction factor” to the length of each stream that was not surveyed.  The correction factor will be the number of redds per kilometer as observed in the random survey of non-index reaches for that year:

Nt,x = Ni,x +(Nn,x/(Pn,x)

Where:

Nt,x = Estimate of the total number of redds in stream x;

Ni,x = Number of redds observed in index reaches of stream x;

Nn,x = Number of redds observed in non-index reaches of stream x; and

Pn,x = Proportion of non-index kilometers surveyed in stream x.

Spring Chinook salmon survey data (collected in August through early October) will be reported in the fall.  Steelhead survey data (collected in April through early June) will be compiled and reported in the early summer.

Pre-Spawn Mortality

Pre-spawning mortality of wild spring Chinook salmon adults upstream of the WSNFH has increased in the past decade.  However, this number is based upon index redd counts and escapement past the hatchery, an average of 5.1 fish per redd was estimated between 2000 and 2007, compared to a ratio of 3.3 fish per redd between 1990 and 1999.  The causes for this increase in pre-spawning mortality are unknown, but this information is important for managing spring Chinook salmon in the watershed.  These ratios were derived by dividing the total number of fish passed at the WSNFH by the number of redds counted in the upper watershed.

The prevailing theory regarding seasonal distribution of spring Chinook salmon in the Warm Springs River is that after they pass the WSNFH between April and June, they hold in deep pools in the Warm Springs Canyon during the summer months before making the migration into the spawning ground.  The Warm Springs Canyon extends approximately 30km from the WSNFH at Rkm 17 upstream to the mouth of Badger Creek at Rkm 47, and includes the lower reaches of Mill and Beaver Creeks.  This canyon is as much as 1,000 feet deep, and is difficult to access, as there is only one road access point in this portion of the River (at Rkm 21.5, about 4.5 km upstream of the WSNFH).

Crews will survey the Warm Springs River Canyon, either by kayak and on foot, two times each year during the summer holding season (once each in July and August).  Crews will look for carcasses of spring Chinook and will collect data on origin, size, and whether the fish successfully spawned.  Additional biological information (e.g. tissue samples to determine presence of disease or parasites) may also be collected.

During spawning surveys in September and October, surveyors will count carcasses encountered during spawning surveys, and will collect data on species, origin (hatchery or wild), size, and whether the fish successfully spawned.

Juvenile Rearing

Beaver Creek, Mill Creek and the Warm Springs River within the Warm Springs River watershed and Shitike Creek will be sampled annually via snorkeling during July and August to collect information on rearing densities of juvenile salmon and steelhead.  The accessible portions of these streams have been segmented into reaches (two in Mill Creek, five in Beaver Creek, six in the Warm Springs River, and six in Shitike Creek) based on geomorphologic differences from above the upper-most reaches of spawning activity to the stream mouths.  Data collected will be used in combination with redd survey and adult spawning escapement data to identify areas of high or low productivity, and provide information on annual trends of productivity.

Reach Selection and Sampling Strategy

A generalized random tessellation stratified (GRTS) sample design (Stevens and Olsen 2003, 2004; Moberg and Ward 2008) will be adapted to select sampling sites.  Following the recommendations of Moberg and Ward (2008), a 10-year study will be initiated beginning in 2011 (Table 18).  A total 180 sample sites and 180 oversample sites will be identified using the GRTS methodology.  The oversample sites are selected in case some of the sample sites are not suitable.  The primary reason that a site would not be suitable for sampling is inaccessibility, as there are reaches in deep canyons with no road access.  From the 180 sample sites identified using the GRTS methodology, a panel of 36 sites (2 from each of the 18 reaches) will be selected for annual sampling for trends status.  Another five rotating panels of 36 sites (2 from each of the 18 reaches) will be selected, with one of the five sampled each year, for a total of 72 sites (4 in each of the 18 reaches) sampled annually.

All sites selected via the GRTS methodology will be sampled during daylight hours between July 1 and September 15, when streams are at their summer base flows.  Reaches lower in the drainages will typically be sampled first, with the higher elevation reaches surveyed in August and September.  In addition to the daylight surveys at the GRTS chosen sites, we will continue night surveys in the eight pools (two in each stream) surveyed since 2006.

Sampling will be stratified so that 50% of the sites selected are pools, and the other 50% are pools and associated riffles.  This will result in 72 pools and 36 riffles being sampled annually.  Crews will travel to the selected location as identified (either on a map or to a GPS point) and move downstream to the first pool.  That pool (and associated riffle, if included) will construe the “site”.  Whether the riffle sampled is upstream or downstream of the pool will be randomly determined.  The entire riffle will be sampled until the next pool is reached up to a maximum of 50m.

Table 18.  Annual panel and rotating panel design of snorkel sites to be sampled (adapted from Moberg and Ward 2008).  Shading indicates the years in which sites in each panel are sampled.

Field Methods

Habitat information collected for each pool and riffle sampled will include: unit length and width at maximum depth (using a measuring tape or electronic range finder), maximum depth and depth at pool tail crest (using a depth staff), number of pieces of large wood within bankfull channel (>3 m in length, > 10 cm at small end, root wads may be less than 3 m), a categorical measure of percent undercut bank (0-25%, 25-50%, 50-75%, 75-100%), and water temperature prior to and after sampling.  All habitat measurements will be measured, not estimated.

Snorkeling will commence at a minimum of 15 minutes after habitat measurements being completed.  Snorkeling methods will be adapted from Thurow (1994) and Murdoch and Nelle (2008).  Block nets will be placed at the upstream and downstream ends of each unit, as stream size allows; placement of block nets in the Warm Springs River may prove difficult.  A team of two or four divers, depending on stream size, will start at the bottom of each unit and move upstream, counting fish by species and size class.  Size classes will be defined as 0 = 0-75mm, 1 = 76 - 150 mm, 2 = 151 - 200 mm, 3 = 201 - 250 mm, and adult = 251mm and above.  Size classes are based on the typical size range of age 1 juvenile spring Chinook salmon in CTWSRO streams (75-150 mm; CTWSRO unpublished data).  One snorkeling pass will conducted on each unit.

Calibration surveys - Although snorkel surveys are among the most cost efficient ways of sampling fish populations, snorkelers tend to undercount fish, and a second method of sampling is required to determine the relationship (correction factor) between visual counts and true counts (Hankin and Reeves 1988; Dollof et al. 1993).  Dollof et al. (1993) suggested that 10% of the diver-sampled units be re-sampled.  Eight sites will be subsampled (2 sites in each of the 4 streams, or 11% of the units sampled) with a second method of sampling to determine true count.  The secondary method of choice will be multiple-pass depletion electrofishing (Temple and Pearsons 2007) which can be implemented more readily in the smaller streams where block nets can be placed (Beaver Creek, Mill Creek, and Shitike Creek).  For the Warm Springs River, which is too large to effectively conduct multiple-pass depletion electrofishing efforts, a mark-recapture electrofishing protocol (Temple and Pearsons 2007) will be implemented.  Preferably, calibration surveys will be conducted on the same day as snorkel surveys, but will at a minimum be conducted within the same week.

Data Analysis

Population estimates for each species will be generated in each of the habitat units sampled.  The total number of fish of each species and size category will be summed across all divers for each unit.  That number will then by expanded by a correction factor determined by the calibration surveys utilized on that particular stream.  Correction factors for each stream will be determined by using the average of the two calibration surveys conducted on that stream (Dollof et al. 1993).  The aforementioned sampling strategy will aid in the analysis of data in a balanced analysis of variance (ANOVA) design for comparisons among reaches: 4 pools and 2 riffles in each of 18 reaches.  The comparison among streams will be unbalanced (two reaches in Mill Creek, five each in Shitike Creek and Beaver Creek, and six in the Warm Springs River).

Once population estimates have been developed for each unit, analysis will be conducted to: 1) investigate differences in fish densities between and among the 18 reaches and 4 streams; 2) investigate relationships between fish densities and data collected habitat variables (utilizing ANOVA for categorical variables and linear regression for continuous variables); and 3) compare annual results to previous years’ data.  Summaries of other fish species observed (e.g. bull trout) will also be reported.

Juvenile Outmigration

Two rotary screw traps will be operated near the mouths of the Warm Springs River (8-foot trap, approximately Rkm 1.5), and Shitike Creek (5-foot trap, approximately Rkm 0.5) in the spring and fall to estimate the number of outmigrating juvenile spring Chinook salmon and steelhead.  The outmigration of both species occurs primarily during high spring flows, typically from March through June (CTWSRO, Unpublished Data).  A smaller outmigration or redistribution of pre-smolt juvenile steelhead and spring Chinook salmon occurs in the fall (Cates 1992).

Field Methods

The traps will be operated from approximately February 15 to June 15 and October 1 to December 15.  During spring, traps will be installed as soon as practicable, depending on ice and snow conditions, and will operate until water temperatures reach 20˚C (68˚F) or until fish show visible signs of stress during trap checking.  Traps will be re-installed and activated on or about the first week of October, when water temperatures decrease, and will be operated until approximately the second week of December or until high flows or ice preclude operation.  Traps will be operated and checked 7 days a week.

Fish handling protocols will be adapted from those outlined by (Tussing 2009).  All fish collected from the traps will be identified according to species and counted.  A random sample of up to 30 fish of each species on each day will be anesthetized using MS-222, measured and weighed, and allowed to recover before release.  To estimate trap efficiency, up to 369 individual age 1 to 3 juvenile fish of each species and age category will be marked and released during the first four days of each week (see Sample Size Determination).  Fish will be marked with a fin clip (upper or lower caudal on a rotating weekly basis) and released approximately 1 km upstream of the trap.  Age 0 fish, provided there is a sufficient number to satisfy sample size requirements, will be marked with a Bismarck brown dye (Gaines and Martin 2004), and similarly released upstream.  Recaptured individuals in the days following release will be counted for trap efficiency estimates but not measured or weighed.

A live box will be constructed for survival and mark retention estimates and a delayed release device (described in Miller et al. 2000) will be constructed to allow for the release of marked fish after dusk.  Additionally, we will investigate methods to increase trap efficiency (e.g. in-stream fencing, redistribution of substrate, etc.).

Information will also be collected on-site for trap revolutions per minute (daily during trap checks) and water temperature (via a data logger placed on site).  Stream flow (discharge) information will be collected from the U.S. Geological Service (USGS) gauging stations located at Rkm 7.1 on the Warm Springs River, and at Rkm 1.0 on Shitike Creek.

Sample Size Determination

The minimum number of fish of each species and age that must be marked and released for estimates of trap efficiency depends on target precision.  The target goal is to be within the half-width of the 95% confidence interval for each stratum.  To determine the standard error (SE) for various trap efficiencies and number of fish released, the following formula will be used:

SE = √(P * (1-P) / R)

Where

P = trap efficiency; and

R = number of fish released (R. Townsend, Univ. of Washington, Columbia Basin Research, Personal Communication).

The half-width of the 95% confidence interval is estimated as

SE * 1.96

A binomial distribution of the precision estimate is assumed, which results in a higher number of tagged fish needed at higher trap efficiencies (Table 19).

Trap efficiencies for juvenile spring Chinook salmon smolts from 2007 through 2009 ranged from 3.4% to 13.0% (mean 7.1%) for the Warm Springs River, and from 18.2% to 36.8% (mean 28.5%) for Shitike Creek.  Juvenile steelhead (age 1, 2, and 3) trap efficiencies ranged from 1.6% to 8.9% (mean 5.9%) for the Warm Springs River, and from 10.0% to 32.0% (mean 23.4%) for Shitike Creek.  Age 0 fish (smaller than 50mm) of both species are generally captured in low numbers and have not been marked for recapture.

Downstream migrant trapping protocols from Tussing (2009) recommend that for migrant traps with efficiencies of less than 7%, a minimum of 100 fish be marked and released for each stratum, which equates to a precision of 5.00%.  The goal will be to mark and release a minimum of 100 fish of each species and age class per stratum.  However, based upon past data, it is likely that there will be weeks in which not enough fish will be captured to meet this goal.  In those cases, a modeled trap efficiency design (Volkhardt et al. 2007) will be utilized to construct trap efficiencies for weeks with insufficient releases.  If environmental conditions (specifically temperature and flow) are similar, weeks may be lumped together (i.e. combined strata) to meet requirements for the targeted number of released fish or substitute trap efficiencies from weeks with similar conditions and a sufficient number of released fish.

Data Analysis

The data collected at the screw traps will be used to generate estimates of the number of juvenile spring Chinook salmon and steelhead outmigrating from the Warm Springs River and Shitike Creek for the spring and fall trapping periods.  A stratified mark-recapture methodology as described in Carlson et al. (1998) will be used to generate population estimates for each stratum.

Table 19.  Minimum numbers of fish to mark and release at various trap efficiencies to be

within the half-width of the 95% confidence interval.

| ## Trap efficiency | ## Minimum number of fish per stratum | ## SE | ## 95% confidence interval half-width |
| --- | --- | --- | --- |
| 3% | 44 | 0.02543 | 4.98% |
| 5% | 73 | 0.02121 | 5.00% |
| 7% | 100 | 0.02551 | 5.00% |
| 10% | 139 | 0.02545 | 4.99% |
| 15% | 196 | 0.02545 | 4.99% |
| 20% | 246 | 0.02550 | 5.00% |
| 25% | 288 | 0.02552 | 5.00% |
| 28% | 310 | 0.02550 | 5.00% |
| 30% | 323 | 0.02550 | 5.00% |
| 35% | 350 | 0.02550 | 5.00% |
| 40% | 369 | 0.02550 | 5.00% |

PIT-Tagging Juvenile Outmigrants

The objective of this PIT tagging program is to investigate the juvenile outmigration patterns and eventual adult returns of spring Chinook salmon and steelhead from and to the Warm Springs River and Shitike Creek, and make the data available to other fish managers in the Columbia River Basin.  This effort will be initially designed to determine if it is feasible to address the following information needs:

- Investigate travel/migration timing of spring Chinook salmon and steelhead outmigrants from the Warm Springs River and Shitike Creek to Bonneville Dam and the Columbia River Estuary.

- Estimate survival rates of spring Chinook salmon and steelhead outmigrants from the Warm Springs River and Shitike Creek to Bonneville Dam and the Columbia River Estuary.

- Compare travel time and survival to hatchery spring Chinook salmon PIT tagged at the WSNFH.

- Investigate relationships between fish measurements at time of tagging (e.g. length, weight, and date) to smolt-to-adult return rates.

- Compare smolt-to-adult return rates of Warm Springs River and Shitike Creek spring Chinook salmon to fish from the WSNFH.

- Investigate the proportion of outmigrating O. mykiss that are visually identified as steelhead smolts at the screw traps that actually display anadromy and migrate out of the Deschutes Subbasin.

- Investigate straying rates of wild spring Chinook salmon and steelhead from the Deschutes Subbasin.  It is known that strays from other systems come into the Deschutes; but do Deschutes fish stray elsewhere?

- Investigate iteroparity (kelts/repeat spawners) in wild steelhead populations in the Warm Springs River or Shitike Creek.

- Compare smolt-to-adult return rates of Warm Springs River and Shitike Creek steelhead to PIT-tagged fish from Round Butte Hatchery.

Field Methods

PIT tags will be applied to juvenile spring Chinook salmon and steelhead > 75mm.  Protocols will be adapted for handling and PIT-tagging wild fish as described for outmigrant traps by Terraqua (2008) and Tussing (2009).  PIT tagging data will be uploaded weekly into the PITAGIS Regional data base.

Sample Size Determination

The Upper Columbia Monitoring Strategy (Hillman 2006) recommends a minimum tagging effort of 5,000 spring Chinook salmon or steelhead smolts to provide sufficient precision for survival estimates.  However, this is a rough estimate, and the actual number of tagged fish needed for adequate precision depends on the survival of tagged fish to detection sites (which must be estimated) and detection efficiency at the sites.

The Program Sample Size (UW - CBR, www.cbr.washington.edu) was used to estimate the precision of survival estimates for hatchery spring Chinook salmon PIT-tagged at the WSNFH in 2007, 2008, and 2010 (data from CBR-UW Data Access in Real Time (www.cbr.washington.edu/dart/.)  Data from 2009 were insufficient to run the analysis. Welch et al. (2008) estimated survival rates from the Snake River to the Bonneville Dam to be between 26.6% and 61.2% (mean 47.0%) for spring Chinook salmon smolts and between 3.8% and 46.2% (mean 33.5%) for steelhead.  Because the Deschutes Subbasin is closer to Bonneville Dam, an estimate of 60% survival rate was chosen for our modeling efforts.  Precision of survival estimates was modeled based on the number of fish tagged and released at the WSNFH, detections at Bonneville Dam, and detections in the Columbia River PIT tag trawls (Table 20).

The highest precision of survival estimates occurred in 2010 (Table 20).  Based upon the 2010 parameters, approximately 23,000 fish of each species would need to be tagged at each of the outmigrant traps to reach a half-width precision of 0.1 at the 90% confidence interval.  However, the numbers of juvenile spring Chinook salmon and steelhead captured at the outmigrant traps from 2007 through 2009 were far below the number needed to obtain such high precision (Table 21).  Precision obtained by tagging a realistic number of fish at the outmigrant traps is much lower (Table 22).

Table 20.  Number of hatchery spring Chinook salmon released from the Warm Springs National Fish Hatchery and detected at the Bonneville Dam and Columbia River Estuary, and the associated half-width precision of survival estimates at 90% and 95% confidence intervals (CIs).  Number tagged in 2008 is approximate.

| Year | ## Number Tagged | ## Number detected at Bonneville Dam | ## Number Detected in Estuary Trawls | ## 95% CI half-width | 90% CI half-width |
| --- | --- | --- | --- | --- | --- |
| 2007 | 19,937 | 684 | 87 | 0.447 | 0.375 |
| 2008 | 15,000 | 1320 | 101 | 0.282 | 0.236 |
| 2010 | 14,907 | 2052 | 209 | 0.147 | 0.124 |

Although the numbers of fish that can be tagged at the outmigrant traps will be insufficient to generate a high precision for survival, the data generated from PIT tagging efforts can be used to investigate some of the information needs presented above.  All results will be reported with the appropriate level of precision, and will include a discussion of the low level of precision and a caveat as to the inferences that may be drawn from the results.

Table 21.  Numbers of juvenile spring Chinook salmon and steelhead captured at the Warm Springs River and Shitike Creek outmigrant traps, 2007 – 2009.

|  |  | ## Spring Chinook Smolts |  |  | ## Steelhead >75mm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trap | Year | Spring | Fall | Total | Spring | Fall | Total |
| Warm Springs River | 2007 | 577 | 1,793 | 2,370 | 343 | 14 | 357 |
|  | 2008 | 1,739 | 1,494 | 3,233 | 361 | 15 | 376 |
|  | 2009 | 430 | 2,001 | 2,431 | 212 | 37 | 249 |
| Shitike Creek | 2007 | 21 | 693 | 714 | 1,425 | 1,112 | 2,537 |
|  | 2008 | 325 | 418 | 743 | 1,057 | 109 | 1,168 |
|  | 2009 | 114 | 1,476 | 1,590 | 554 | 155 | 709 |

Table 22.  95 and 90% confidence intervals (CIs) resulting from tagging between 1000 and 4000 fish.

| ## Number of fish tagged | ## 95% CI half-width | ## 90% CI half-width |
| --- | --- | --- |
| 1000 | 0.560 | 0.447 |
| 1500 | 0.464 | 0.389 |
| 2000 | 0.403 | 0.338 |
| 2500 | 0.360 | 0.302 |
| 3000 | 0.329 | 0.276 |
| 3500 | 0.304 | 0.255 |
| 4000 | 0.285 | 0.239 |

Goal 2) Provide management and co-management direction of the fisheries resources in the Deschutes River Subbasin.

The CTWSRO exclusively manages fisheries resources on Reservation lands, and, cooperatively manages fisheries on all Tribal ceded lands with the Oregon Department of Fish and Wildlife (ODFW).  Ceded lands include the Deschutes River, John Day River, and Hood River subbasins.  Other agencies and research partners in the Deschutes River Subbasin include the USFWS, US Forest Service, Bureau of Land Management, Portland General Electric, and local irrigation districts, watershed councils and public interest groups.  The CTWSRO cooperates and coordinates with these groups, and attends meetings to share information and stay informed on fisheries issues in the Deschutes Subbasin, and to ensure Tribal interests and policies are heard and understood.

Although some of the information used to manage fish resources in the Deschutes River Subbasin will be collected through work conducted as part of this proposal, the weighing of that information by co-managers, and the actual management decisions are not part of this proposal.  These management activities are policy in nature rather than technical.  It is therefore appropriate that we limit our description to a general overview of management activities.

In addition to collecting, analyzing, and reporting information as described for Goal 1, the success of Goal 2 relies on effective coordination and cooperation by the CTWSRO with co-managers, primarily ODFW and the USFWS.  Effective coordination and cooperation requires communication for accurate and timely information exchange.  Each objective within this goal (cooperating in fisheries management activities, estimating harvest, and providing co-management and assistance at WSNFH) can be considered successful only if key indicators or metrics of impact are established and met.  These key indicators include:

- Benefits to the fish populations affected (informed management decisions result in healthier populations);

- Benefits to potential users – both tribal and non-tribal (effective regulations that minimize conflict and facilitate health of the populations); and

- Continued good working relationships among co-managers (primarily CTWSRO, ODFW, and the USFWS).

The specific objectives of providing management and co-management direction include:

- 1) Cooperating in Deschutes River Basin Fisheries Management Activities;

- 2) Estimating harvest of Chinook salmon and Steelhead in the Deschutes River Subbasin;

- 3) Providing co-management and assistance with fish handling at the WSNFH.

Cooperate in Deschutes River Basin Fisheries Management

Actively cooperating in fisheries management activities is an essential part of meeting salmon recovery goals in the future.  Work conducted during activities described under Goal 1 provides data on the numbers and trends of the life stages of spring Chinook salmon and steelhead in the Deschutes Subbasin.  However, those data must be analyzed and used to make informed decisions about land management, habitat restoration, harvest regulations and seasons, potential supplementation, and other issues.  Management activities that must be addressed annually include development of preseason escapement forecasts, setting harvest seasons and regulations, managing license sales and distribution, and providing enforcement of regulations.

Each year, co-managers complete a reconstruction of the previous year's spring Chinook salmon run, and use regression and cohort models to predict the run of wild and hatchery adults to the Round Butte Hatchery, the WSNFH, and the Warm Springs River.  This prediction is cooperatively developed between the CTWSRO, USFWS, and ODFW.  The co-managers analyze results from various models, including point estimates and confidence intervals, and then use this information along with professional judgment to estimate run size.  Harvest regulations are primarily driven by the estimate of wild fish.

Steelhead harvest regulations have been consistent for tribal fisheries in the Deschutes River since Mid-Columbia steelhead were listed as threatened under the ESA.  No limit is placed on the number of hatchery-origin fish taken, but all wild fish must be released. Lack of a reliable method to estimate steelhead run size has precluded any changes in regulations.

In general, all areas within the ceded lands are open to harvest by Tribal Members for all species of fish, unless restricted by Tribal Resolution (except for Columbia River Zone 6 fisheries, which are regulated by the US v Oregon agreement).  Harvest regulations are set to restrict for conservation purposes (e.g. requiring release of wild spring Chinook salmon and steelhead).

Estimate Harvest of Spring Chinook Salmon and Steelhead in the Deschutes Subbasin

As described above, the CTWSRO and ODFW generate preseason estimates of adult spring Chinook salmon escapement to the Deschutes River.  These estimates rely heavily on run reconstruction of the previous year’s escapement, and are used directly when developing regulations for sport and Tribal harvest in the Deschutes River.

Harvest in the Deschutes River is focused at and around the Sherars Falls area (Rkm 70.4).  Annual creel surveys conducted by ODFW and CTWSRO creel clerks at Sherars Falls provide this harvest information.  Data are collected on numbers of fishermen, effort (hours), technique (scaffold or hook and line) and the numbers of fish harvested.  Further information on harvested fish includes origin (wild or hatchery), size (for age estimation), and any other distinguishing marks (e.g. fin clips or floy tags).  Creel surveys and analysis of creel survey data will not be funded through this proposal; therefore, more detail on methods and analyses are not included here.

Provide Co- Management and Assistance with Fish Handling at the

Warm Springs National Fish Hatchery

The USFWS and the CTWSRO co-operatively manage the WSNFH, which is located on Tribal land leased to the USFWS.  The operation of the WSNFH is operated under a co-operative Operational and Implementation Plan that is updated every five years.  The current document expires in 2011 (CTWRSO and USFWS 2007), and the CTWSRO will take the lead in writing the new 2012 – 2017 document.

Staff from the CTWSRO, along with staff from the USFWS, serves on the Hatchery Evaluation Team (HET) for the WSNFH.  The HET meets quarterly to receive updates on hatchery operations, evaluate and comment on proposed studies, and disseminate information on existing or completed studies.  Additionally, the CTWSRO and the USFWS alternately host an annual meeting for presentation of study results and hatchery operations.  This annual meeting also provides an opportunity for members of the Tribal and non-tribal public, staff from other agencies in the Deschutes Basin, and Tribal Council and Committee members to learn about operations at the WSNFH.

Although the WSNFH is primarily a production facility, it is also used as a tool to improve and enhance runs of wild spring Chinook salmon and steelhead.  Management actions at the hatchery are used to meet the long-term Deschutes Subbasin goal of achieving a spawning escapement of 2,200 to 2,300 adult wild spring Chinook salmon above the barrier dam at the WSNFH (NPCC 2005).

When the WSNFH has a large number of steelhead and/or Chinook salmon to process, CTWSRO staff may be called upon to assist with fish sorting, handling, and distribution to Tribal Membership, or to assist with other hatchery operations.  Although the spring and summer months (April – September) are the busiest months of the year in terms of fish handling, CTWSRO staff may be requested to assist in operations at any time of the year.  The CTWSRO and the USFWS have an annual cooperative agreement that supplies partial funding to the CTWSRO Fisheries Program for these duties.

FACILITIES AND EQUIPMENT

Facilities, personnel, equipment, and vehicles will be provided by the CTWSRO.  New equipment purchases with BPA funds will include new computers, analysis software,  office supplies, PIT tags and PIT tag readers, field gear including waders, uniforms polarized glasses, and dive gear (masks and snorkels), and digital cameras.  Other equipment purchases in the future may include gear necessary for data collection, including waders, dry suits and snorkeling gear, inflatable kayaks and paddles, polarized sunglasses, nets, chemicals for fish handling, and hand and power tools.

The costs for portions of three GSA vehicles will be charged to this budget annually.  Other costs include lodging, per diem and travel to Oregon AFS conferences and various meetings, and subcontracts for repair and maintenance of rotary screw traps and other services.

REFERENCES

Carlson, S. R., L. Coggins, and C. O. Swanton. 1998. A simple stratified design for mark–recapture of salmon smolt abundance. Alaska Fisheries Research Bulletin 5(2): 88–102. Available: www.adfg.state.ak.us/pubs/afrb/vol5_n2/carlv5n2.pdf (August 2005).

Cates, B. 1992.   Warm Springs National Fish Hatchery Evaluation and Anadromous Fish Study on the Warm Springs Indian Reservation of Oregon, 1975 – 1989 Progress Report.  USFWS Lower Columbia River Fisheries Program Office, Vancouver, Washington

CTWRSO and USFWS.  2007.  Warm Springs National Fish Hatchery Operational and Implementation Plan 2007-2011.  USFWS Lower Columbia River Fisheries Program Office, Vancouver, Washington

Dambacher,  J.M.  2002.  Project report:  relative abundance of juvenile Chinook salmon

in Shitike Creek, of the Confederated Tribes of the Warm Springs Reservation,

Oregon.  Oregon Department of Fish and Wildlife, Corvallis Research Lab,

Corvallis, OR.

Dollof, C. A, D.G. Hankin and G.H Reeves. 1993.  Basinwide estimation of habitat and fish population in streams.   U. S. Forest Service, Southeastern Forest Experiment Station, General Technical Report SE-83.  Asheville, North Carolina

Engle, R. and D. Olsen.  2006.   Abundance and Microhabitat Selection of Juvenile Oncorhynchus mykiss and Juvenile Chinook Salmon (O. tshawytscha) Within Shitike Creek, Oregon at Varying Fish Densities and Presence or Absence of Bull Trout (Salvelinus confluentus). Progress Report Fiscal Year 2005 and 2006 Work Plan.  US Fish and Wildlife Service, Columbia River Fisheries Program Office, Vancouver, Wa

Gaines, P. C, and C. D. Martin.  2004.  Feasibility of Dual-Marking Age-0 Chinook Salmon for Mark–Recapture Studies. North American Journal of Fisheries Management 24: 1456-1459

Gallagher, S. P.,  P. K. J. Hahn, and D. H. Johnson.  Redd Counts.  Pages 197-233 in: Johnson, D. H, B. M. Shrier, J. S. O’Neal, J. A. Knutzen, X. Augerot, T. A. O’Neil, and T. N. Pearsons.  2007.  Salmonid field protocols handbook: techniques for assessing status and trends in salmon and trout populations.  American Fisheries Society, Bethesda, Maryland.

Hankin, D.G., and G.H Reeves.  1988.  Estimating Total Fish Abundance and Total Habitat Area in Small Streams Based on Visual Estimation Methods.  Canadian Journal of Fisheries and Aquatic Sciences 45: 834-844

Hillman, T. W.  2006.  Monitoring Strategy for the Upper Columbia Basin, Second Draft Report.  Prepared for the Upper Columbia Salmon Recovery Board, BPA and NMFS.  BioAnalysis, Inc.  Boise, Idaho

Johnson, D. H, B. M. Shrier, J. S. O’Neal, J. A. Knutzen, X. Augerot, T. A. O’Neil, and T. N. Pearsons.  2007.  Salmonid field protocols handbook: techniques for assessing status and trends in salmon and trout populations.  American Fisheries Society, Bethesda, Maryland.

Miller, B. A., J. D. Rodgers, and M.S. Solazzi.  2000.  An automated device to release marked juvenile fish for measuring trap efficiency.  North American Journal of Fisheries Management 20: 284-287

Moberg, J, and M. Ward.  2008.  A field Manual of Scientific Protocols for Selecting Sampling Sites used in the Integrated Status and Effectiveness Monitoring Program.  2008 Working Version 1.0.  Prepared for BPA’S ISEMP.  Terraqua, Inc Wauconda, Washington

Murdoch, K., and R. D. Nelle.  2008.  A Field Manual of Scientific Protocols for Underwater Observations within the Upper Columbia Monitoring Strategy.  2008 Working Version 1.0.  Prepared for BPA’S ISEMP.  Terraqua, Inc. Wauconda, Washington

National Marine Fisheries Service, Northwest Region.  2009.  Middle Columbia River Steelhead Distinct Population Segment ESA Recovery Plan.  http://www.nwr.noaa.gov/Salmon-Recovery-Planning/Recovery-Domains/Interior-Columbia/Mid-Columbia/Mid-Col-Plan.cfm

Nelle, R.D., and J. Moberg.  2008.  A Field Manual of Scientific Protocols for Steelhead Redd Surveys within the Upper Columbia Monitoring Strategy.  2008 Working Version 1.0.  Prepared for BPA’S ISEMP.  Terraqua, Inc. Wauconda, Washington

Northwest Power and Conservation Council. 2005. Deschutes Subbasin Plan. In Columbia River Basin Fish and Wildlife Program. Portland, Oregon, 2005.

Northwest Power and Conservation Council, 2009.  Columbia River Basin Fish and Wildlife Program, Portland, Oregon.

Parsons, A. L., and J. R. Skalski.  2009.  The design and analysis af salmonid tagging studies in the Columbia Basin, Volume XXIV.  A statistical critique of estimating salmon escapement in the Pacific northwest.  Prepared for USDE BPA, Project No 1989-107-00.  School of Aquatic and Fishery Sciences, University of Washington, Seattle, Washington.

Stewart, R.  2002.  Resistance board weir panel construction manual.  Alaska Department of Fish and Game, Division of Commercial Fisheries, Arctic-Yukon-Kuskokwin Region, Regional Information Report No. 3A02-21, Anchorage, AK

Stewart, R. 2003.  Techniques for installing a resistance board fish weir.  Alaska Department of Fish and Game, Division of Commercial Fisheries, Arctic-Yukon-Kuskokwin Region, Regional Information Report No. 3A02-21, Anchorage, AK

Stevens, D.L. Jr. and A. R. Olsen.  2003. Variance Estimation for Spatially Balanced Samples of Environmental Resources.  Environmetrics 14: 593-610

Stevens, D.L. Jr. and A. R. Olsen.  2004.  Spatially Balanced Sampling of Natural Resources.    Journal of the American Statistical Association 99:262-278

Temple, G. M. and T. N. Perasons.  2007.  Electrofishing: Backpack and Drift Boat.  Pages 95-132 in Johnson, D. H, B. M. Shrier, J. S. O’Neal, J. A. Knutzen, X. Augerot, T. A. O’Neil, and T. N. Pearsons.  2007.  Salmonid field protocols handbook: techniques for assessing status and trends in salmon and trout populations.  American Fisheries Society, Bethesda, Maryland.

Terraqua, Inc.  2008.  A Field Manual of Scientific Protocols for Captured, Handling, and Tagging of Wild Salmonids in the Upper Columbia River Basin using Passive Integrated Transponder (PIT) tags within the UpperColumbia Monitoring Strategy.  2008 Working Version 1.0.  Prepared for BPA’S ISEMP.  Terraqua, Inc Wauconda, Washington

Thurow, R. F. 1994.  Underwater methods for study of salmonids in the intermountain west.  U. S. Forest Service, Intermountain Research Station, General Technical Report INT-GTR-307.  Ogden, Utah

Tussing, S. P.  2009. A Field Manual of Scientific Protocols for Downstream Migrant Trapping within the Upper Columbia Monitoring Strategy.  2009 Working Version 1.0.  Prepared for BPA’S ISEMP.  Terraqua, Inc. Wauconda, Washington

Townsend, Rich. 2010. Columbia Basin Research, University of Washington. Personal Communication.

Volkhardt, G. C., S. L. Johnson, B. A. Miller, T. E, Nickelson, and D E. Seiler.  2007.  Rotary Screw Traps and Inclined Plane Screen Traps.  Pages 235 - 266 in: Johnson, D. H, B. M. Shrier, J. S. O’Neal, J. A. Knutzen, X. Augerot, T. A. O’Neil, and T. N. Pearsons.  2007.  Salmonid field protocols handbook: techniques for assessing status and trends in salmon and trout populations.  American Fisheries Society, Bethesda, Maryland.

Welch D.W., Rechisky EL, Melnychuk MC, Porter AD, Walters CJ, et al.  2008. Survival of Migrating Salmon Smolts in Large Rivers With and Without Dams. PLoS Biol 6(10): e265. doi:10.1371/journal.pbio.0060265

Zimmerman, C. E., and L.M. Zabkar.  2007.  Weirs.  Pages 385-397 in: Johnson, D. H, B. M. Shrier, J. S. O’Neal, J. A. Knutzen, X. Augerot, T. A. O’Neil, and T. N. Pearsons.  2007.  Salmonid field protocols handbook: techniques for assessing status and trends in salmon and trout populations.  American Fisheries Society, Bethesda, Maryland.

Key Personnel:

Jens Lovtang, Fisheries Management Supervisor

Principal Investigator/Project Manager

Education

Oregon State University, M.S. Fisheries Biology, 2005

Humboldt State University, B.S Natural Resources Planning, 1995

Work Experience

Confederated Tribes of Warm Springs

Fisheries Management Supervisor, October 2009 – Present

Fish Production Biologist, January 2006 – October 2009

Harvest Manager, November 2005 – December 2005

Oregon Department of Fish and Wildlife, Corvallis, Oregon

Experimental Biology Aide, Gearhart Mountain Bull Trout Project, July – August 2005

Experimental Biology Aide, Siletz River Fall Chinook Project, September - October 2005

Oregon State University, Corvallis, Oregon

Graduate Research Assistant (M.S. Candidate), January 2002 – June 2005

Portland General Electric, Madras, Oregon

Fish Technician, Pelton Round Butte Project April – November 1999

Deschutes National Forest, Sisters Ranger District, Sisters, Oregon

Seasonal Fisheries Biologist, 1996 – 1998

Recent Publications

Confederated Tribes of the Warm Springs Reservation of Oregon and The United States Fish and Wildlife Service.  2007.  Warm Springs National Fish Hatchery, 2007-2011 Operational Plan and Implementation Plan.  USFWS, Columbia River Fisheries Program Office, Vancouver, WA.

Lovtang, J.C.  2008. Confederated Tribes of Warm Springs, Fish Production Program, 2007 Annual Report.  Confederated Tribes of Warm Springs, Warm Springs, OR

Lovtang, J.C., M. Hill, R. Stocking, and B. Hodgson.  2008.  Lake Billy Chinook/Metolius River 2007 Kokanee Spawning Population Studies. Tab 17 in Portland General Electric Pelton Round Butte Project, 2008 Annual Fisheries Workshop.  Portland General Electric, Portland, Oregon.

Lovtang, J.C., 2008.  Chinook Reintroduction/Fry Releases in the Metolius River Basin, 2008.  Tab 12a in Portland General Electric Pelton Round Butte Project, 2008 Annual Fisheries Workshop.  Portland General Electric, Portland, Oregon.

Lovtang, J.C., P. Galbreath, and S. Hyun. 2007.  Using a Dual-Frequency Identification Sonar (DIDSON) to enumerate the spawning migration of Kokanee salmon into the Metolius River.  Tab 19 in Portland General Electric Pelton Round Butte Project, 2007 Annual Fisheries Workshops.  Portland General Electric, Portland, Oregon.

Lovtang J.C.  2005.  Distribution, Habitat Use, and Growth of Juvenile Chinook Salmon in the Metolius River Basin, Oregon.  Masters's Thesis. FERC Number 2030. Portland General Electric, Portland, Oregon.

Lisa Hewlett-Dubisar, Fisheries Biologist

Project Implementation

Education:

Portland State University, B.S Environmental Science, 2000

Work Experience

The Confederated Tribes of Warm Springs

Fisheries Biologist, November 2005 - Present

Make recommendations and mitigations to conserve, protect, and enhance fisheries resources.  Apply the Tribal Integrated Resource Management Plan to timber sales and other land use practices.  Write biological assessments on ESA listed fish species.  Review proposed projects and write biological evaluations.  Frequently communicate with various state and federal agencies.  Conduct spring Chinook, steelhead, and bull trout redd counts.  Use GPS and GIS to geographically present field data.  Construct a fish weir to collect summer steelhead kelts and to monitor underwater video to enumerate upstream migrating adult salmonids.  Present project results in technical reports, maps, and professional presentations.

Oregon Department of Fish and Wildlife

Experimental Biology Aid,

February 2005 – October 2005; June 2004 – October 2004; June 2001 – October 2001;

Performed various sampling techniques to determine escapement and productivity of spring Chinook and summer steelhead in the John Day subbasin.  PIT tagged emigrating smolts using rotary screw traps and 100ft beach seine.  Performed Chinook, steelhead, and bull trout spawning ground surveys.  Identified fin marks, sex, origin, collect scale, kidney, genetic, and ovary samples from salmonid carcasses.  Operated and navigated drift boat and jet sled down the Mainstem John Day River.

Department of Environmental Quality

Natural Resource Specialist 1, June 2004 – October 2004

Oregon State University

Research Assistant, February 2001 – June 2001

JOB-RELATED PUBLICATIONS:

Hewlett, L.M.  2007.  The Confederated Tribes of the Warm Springs Reservation of Oregon.  Shitike Creek Summer Steelhead (Oncorhynchus mykiss) Kelt Reconditioning: Reproductive Success, Monitoring, and Assessment.

Lovtang, J., L. M. Hewlett, and A. Mitchell. 2006.  The Confederated Tribes of the Warm Springs Reservation of Oregon Fish Production Program. Annual Progress Report 2006.

Lovtang, J., D. Best, L. M. Hewlett, and A. Mitchell. 2005.  The Confederated Tribes of the Warm Springs Reservation of Oregon Fish Production Program. Annual Progress Report 2005.

Arthur Mitchell, Lead Implementation Technician

Art has worked for the CTWSRO Department of Natural Resources since 2002.  His experience with the CTWSRO Fish Production program includes serving as crew leader for spring and fall migrant trapping, redd surveys, snorkeling efforts, and the Shitike Creek steelhead kelt reconditioning program.  Art coordinates field activities for the program, including snorkeling and redd counts, and oversees data entry for all field data collection.

Education

Graduated from Molalla Union High School

Molalla, Oregon 1986

Professional Experience

Confederated Tribes of Warm Springs, 2002 - present

Lead Technician, Fish Production Program

Duties: Coordinate and implement Tribal fisheries research projects including operation and maintenance of field gear (e.g., picket weirs, migrant traps); maintain equipment; lead daily field crew activities; effectively communicate, orally and written; enter, verify, and assist with summarizing of field data; assist in report preparation; and coordinate field sampling activities with a variety of entities.

Ironworkers Union Local #27, Portland, Oregon

Apprentice Ironworker 2000-2002

Yakama Nation, Land Enterprises, Wapato, Washington

Irrigation Field Crew Boss, 1994-2000

________________________________________

w:\mf\ww\moa 2008-2010\three tribes\submittals\nov4 accord 11 narratives\200831100\2011\040111 ctws production narrative 3-29-11.doc