April 16, 2009

TO:		NEEA Board of Directors

FROM:	Claire Fulenwider, Executive Director

SUBJECT:	6th Power Plan and the 2010-2014 Business Plan – FOR YOUR INFORMATION

Background.  As you know, The Northwest Power and Conservation Council is in the final stages of drafting the 6th Northwest Power Plan.  A draft of the 6th Plan is scheduled for release in May with a projected final adoption in August.

Although the numbers are still in review, Council staff are projecting that the 6th Plan will identify a very significant conservation resource potential; perhaps as much as 5,000 aMW to be captured over the next 20 years.  This very large target presents real challenges to the region and to NEEA’s 2010-2014 Business Plan.

Overlay with the 2010-2014 Business Plan.  In order to better understand the challenge presented by the 6th Plan, staff has reviewed the most recent supply curve documents from the Council and mapped them against the markets identified for direct supporting activities in the 2010-2014 Business Plan.  This comparison consisted of the following steps:

- Identify 6th Plan efficiency measures and technically-achievable energy savings potential with cost-effectiveness up to 100 mills/kWh.

- Map these measures against 2010-2014 Business Plan market activities.

- Aggregate savings where the Business Plan provided direct support to the market that delivers the efficiency measure.

For this exercise, no judgment was made regarding what fraction of a specific measure would be supported by Business Plan activities or what entity should take a lead role in securing the savings in these markets.  It is intended to provide a general sense of “coverage” of the key 6th plan efficiency opportunities and to identify early “gaps” in comparison to the 2010-2014 Business Plan.

Summary Outcomes.  The draft data from the Council suggests a total regional technically achievable potential of just under 5,500 average megawatts; roughly double what was identified in the 5th Power Plan. Of this potential, NEEA’s projected activities for the 2010-2014 Business Plan provide support for markets that would affect roughly 3,300 aMWs or just about 60% overall.  However, when split into lost opportunities versus discretionary resources, NEEA provides significantly more support for lost opportunities (79%) and less for discretionary resources (40%).  This is probably appropriate given that discretionary resources tend to be better targets for local utility programs (e.g. weatherization, lighting retrofits).  Table 1 below illustrates the breakdown by resource type.

Table 1.  Summary Mapping of 6th Plan to 2010-2014 BP

|  | 6th Plan Resource Potential <100 mills/kWh | Supported by NEEA 2010-2014 Business Plan | % |
| --- | --- | --- | --- |
| Total Lost Opportunities | 2894 | 2275 | 79% |
| Total Discretionary | 2588 | 1042 | 40% |
| Total | 5482 | 3317 | 61% |

The attachment includes a more detailed breakdown of the comparison by sector and resource type.

Gap analysis.  In order to better understand the gaps between the 2010-2104 Business Plan and the 6th Plan, measures were sorted by total potential into “top 10” lists for each sector separated into lost-opportunity and discretionary resources.  These top-10 measures represent 84% of the total potential in the 6th plan.

In lost opportunity markets, seven of the top-10 residential and 8 of commercial measures are supported by the 2010-2014 Business Plan.  For discretionary resources, only 2 residential and 2 commercial measures are supported.  For industrial, 3 lost opportunity and 6 discretionary top-10 measures are supported.

A more detailed comparison of specific “top 10” measures in the 6th Plan versus support in the current 2010-2014 Business Plan are shown in Table 2 in the Attachment.

Conclusions.  Although brief, this mapping exercise indicates that there is already fairly good alignment between the 6th plan and the 2010-2014 Business Plan.  This is especially true for lost-opportunity resources that are traditionally better targets for upstream focused activities or codes and standards efforts.  This is less true for discretionary markets, although NEEA has targeted some high-priority markets in these markets.

Next Steps.  Staff will be working with BPA in a joint exercise to further understand the details of the supply curves and refine the levels of support included in the Business Plan.  NEEA and BPA will call together a meeting of regional stakeholders to review this effort in May or early June following release of the Draft Plan.  This first meeting will be followed by a second that will discuss likely roles and responsibilities.  NEEA will integrate the outcome of these meetings as one part of the portfolio management process that will be developed over the rest of this year.

Attachment

ATTACHMENT

Figure 1.  6th Plan Technically Achievable Energy Savings Potential by Sector and Resource Type Mapped to 2010-2014 Business Plan Market Support

Table 2a Residential Lost Opportunities - Top 10 Measures

|  | Measure description | Resource Potential < 100 mills/kWh aMW | Supported in 2010-2014 BP ? | Resource Potential Supported by BP aMW |
| --- | --- | --- | --- | --- |
| 1. | Energy Star Televisions | 511 | Yes | 511 |
| 2. | HPWH | 500 | Yes | 500 |
| 3. | Energy Star Residential Desktop Computers | 178 | Yes | 178 |
| 4. | HVAC HP Upgrade | 146 | No |  |
| 5. | Clothes Washer | 110 | Yes | 110 |
| 6. | Energy Star Commercial Desktop Computers | 105 | Yes | 105 |
| 7. | Energy Star Set Top Boxes | 91 | No |  |
| 8. | Shell Upgrade - New SF | 68 | Yes | 68 |
| 9. | Efficient DHW Tanks | 57 | No |  |
| 10. | Refrigerator | 41 | Yes | 41 |
|  | TOTAL | 1806 | 7 | 1512 |

Table 2b Residential Discretionary - Top 10 Measures

|  | Measure description | Resource Potential < 100 mills/kWh aMW | Supported in 2010-2014 BP ? | Resource Potential Supported by BP aMW |
| --- | --- | --- | --- | --- |
| 1. | HVAC HP Conversion | 370 | No |  |
| 2. | HVAC Supplement (DHP) | 235 | Yes | 235 |
| 3. | ResWX SF | 228 | No |  |
| 4. | Lighting | 152 | Yes | 152 |
| 5. | ResWX MF | 135 | No |  |
| 6. | ResWX MH | 104 | No |  |
| 7. | Showerheads | 86 | No |  |
| 8. | Solar DHW | 0 | No |  |
| 9. | Solar PV | 0 | No |  |
|  | TOTAL | 1311 | 2 | 387 |

Table 2c Commercial Lost Opportunities - Top 10 Measures

|  | Measure description | Resource Potential < 100 mills/kWh aMW | Supported in 2010-2014 BP ? | Resource Potential Supported by BP aMW |
| --- | --- | --- | --- | --- |
| 1. | Lighting Power Density-NR* | 290 | Yes | 290 |
| 2. | Exterior Building Lighting-NR | 61 | Yes | 61 |
| 3. | Integrated Building Design-New | 57 | Yes | 57 |
| 4. | Lighting Controls Interior-NR | 52 | Yes | 52 |
| 5. | Packaged Refrigeration Equipment-New | 48 | No |  |
| 6. | Lighting Power Density-New | 42 | Yes | 42 |
| 7. | Parking Lighting-NR | 36 | Yes | 36 |
| 8. | Street and Roadway Lighitng-NR | 29 | Yes | 29 |
| 9. | Roof Insulation-NR | 25 | No |  |
| 10. | Premium Fume Hood-New | 20 | Yes | 20 |
|  | TOTAL | 661 | 8 | 588 |

*NR – denotes “natural replacement” on burn-out or tenant improvement cycles

Table 2d Commercial Discretionary - Top 10 Measures

|  | Measure description | Resource Potential < 100 mills/kWh aMW | Supported in 2010-2014 BP ? | Resource Potential Supported by BP aMW |
| --- | --- | --- | --- | --- |
| 1. | Grocery Refrigeration Bundle-Retro | 122 | No |  |
| 2. | Controls Commission Complex HVAC-Retro | 107 | Yes | 107 |
| 3. | Computer Servers and IT-Retro | 88 | No |  |
| 4. | Network PC Power Management-Retro | 78 | No |  |
| 5. | Municipal Sewage Treatment-Retro | 37 | No |  |
| 6. | Lighting Power Density-Retro | 32 | No |  |
| 7. | Demand Control Ventilation-Retro | 18 | No |  |
| 8. | Package Roof Top Optimization and Repair-Retro | 14 | Yes | 14 |
| 9. | Windows-Retro | 13 | No |  |
| 10. | Municipal Water Supply-Retro | 13 | No |  |
|  | TOTAL | 522 | 2 | 121 |

Table 2e Industrial Lost Opportunities - Top 10 Measures

|  | Measure description | Resource Potential < 100 mills/kWh aMW | Supported in 2010-2014 BP ? | Resource Potential Supported by BP aMW |
| --- | --- | --- | --- | --- |
| 1. | HighBay Lighting 1 Shift | 4 | No |  |
| 2. | HighBay Lighting 2 Shift | 3 | No |  |
| 3. | HighBay Lighting 3 Shift | 23 | No |  |
| 4. | Efficient Lighting 1 Shift | 4 | No |  |
| 5. | Efficient Lighting 2 Shift | 3 | No |  |
| 6. | Efficient Lighting 3 Shift | 24 | No |  |
| 7. | Lighting Controls | 8 | No |  |
| 8. | Motors: Rewind 20-50 HP | 1 | Yes | 1 |
| 9. | Motors: Rewind 51-100 HP | 1 | Yes | 1 |
| 10. | Motors: Rewind 101-200 HP | 1 | Yes | 1 |
|  | TOTAL | 74 | 3 | 3 |

Table 2f Industrial Discretionary - Top 10 Measures

|  | Measure description | Resource Potential < 100 mills/kWh aMW | Supported in 2010-2014 BP ? | Resource Potential Supported by BP aMW |
| --- | --- | --- | --- | --- |
| 1. | Air Compressor Energy Management | 16 | Yes | 16 |
| 2. | Air Compressor Equipment1 | 7 | No |  |
| 3. | Air Compressor Optimization | 19 | Yes | 19 |
| 4. | Efficient Centrifugal Fan | 3 | No |  |
| 5. | Fan Energy Management | 10 | Yes | 10 |
| 6. | Fan Equipment Upgrade | 29 | No |  |
| 7. | Fan System Optimization | 40 | yes | 40 |
| 8. | Pump Energy Management | 11 | Yes | 11 |
| 9. | Pump Equipment Upgrade | 32 | No |  |
| 10. | Pump System Optimization | 26 | Yes | 26 |
|  | TOTAL | 193 | 6 | 122 |

Council staff suggest that marginal costs for screening efficiency measures are likely to be around 100 mills/kWh.