# Habitat Analysis Metadata

Data layers linking NPPC (Streamnet) habitat and CC data to the Streamnet stream layer were obtained from Anne as individual shapefiles for chinook and steelhead.  The chinook data layer had apparently been ‘clipped’ using 6th field HUCs prior to my obtaining it resulting in some reaches being divided into multiple segments where they crossed HUC lines.  This was not true for the steelhead layer.

Data Manipulations/Summary:

- Chinook data layer was summarized by RRN in order to ‘rejoin’ appropriate reach segments into a single observation for each stream reach.

- Fit of the Streamnet stream layer with our HUC layer was visually assessed to be relatively poor, with stream segments intended to lie in a single HUC often having portions crossing into other HUCs.  The degree of error was deemed ‘standard’ as often associated with re-projection and combination of data from different sources.

- Because of the situation described in (2), stream reach layers associated with Habitat Quality and CC for both chinook and steelhead were converted to centroids.  This process associates the data with the central portion of the reach and results in minimization of the influence of mapping errors discussed in (2).

- Habitat quality rankings of Excellent, Good, Fair and Poor were assigned values of 4, 3, 2, and 1, respectively.  Weighted averages of habitat quality were calculated for each HUC containing data using the usable area defined by NPPC as a weighting factor.  This step was conducted separately for each species.

Assumptions Associated with Data Manipulations/Summary:

- Conversion of stream reaches to centroids more accurately assigns data to the appropriate HUC when summarized.  Visual review of centroid locations supports this assumption.